SECTION C

GENERAL CONSTRUCTION PRINCIPLES

Contents

1.	SITE PREPARATION

1.1	Preliminary Investigation

1.2	Check list for Site Conditions

1.3	Site Clearance

1.4	Material Storage

1.5	Batter Boards

1.6	Roads and Pavings

2.	EARTH MOVING

2.1	Excavations

2.2	Backfilling

2.3	Compaction

3.	EARTHQUAKE AND HURRICANE CONSIDERATIONS

3.1	Earthquake Resistant Construction

3.1.1	General

3.1.2	Effect of Soil Type

3.1.3	Effect of High Seas

3.1.4	Building Shape

3.1.5	Appendages

3.2	Rules for Construction of Earthquake Resistant Buildings

3.2.1	Location of Openings

3.2.2	Masonry Buildings

3.2.3	Steel Buildings

3.2.4	Timber Buildings

3.3	Hurricane Resistant Construction

3.3.1	General

3.4	Rules for Construction of Hurricane Resistant Buildings

3.4.1	Building Site

3.4.2	Roofs

3.4.3	Windows and Doors

3.4.4	Walls

3.4.5	Timber Buildings

3.4.6	Steel Buildings

Figures

C-1	Plan Shapes of Buildings

C-2	Size and Location of Wall Openings

C-3	Typical Wall Corner Details

C-4	Typical Wall Junction Detail

C-5	Reinforced Concrete Gable Arrangement

C-6	Recommended Method of Construction on Sloping Sites

C-7	Hip Roof Construction

C-8	Gable Roof Construction

C-9	Rafter/Ring Beam Connection

C-10	Wall Plate Connections and Hurricane Straps

C-11	In-fill Panel between Timber Building Supports

C-12	Timber Framing for Wall

C-13	Timber Framing showing Bracing

C-14       Construction  Site Layout

C-15       Roof  Detail at Parapet Wall

C-16 A&B    Damp-proofing for Retaining Walls to Buildings

SECTION C

GENERAL CONSTRUCTION PRINCIPLES

1	SITE PREPARATION

1.1	Preliminary Investigation

Before any construction work commences, a preliminary inspection of the site shall be undertaken so that preparation may be made for any problems or difficulties that may arise.  This time should also be used to plan how the site will be organised so that a logical layout may emerge. Completion of this checklist will provide enough information about the site and its conditions to permit construction to begin.

1.2	Checklist for Site Conditions

1)	Take note of general topography of site and other physical conditions likely to cause hazards.

2)	Is there adequate natural provision for removal of storm water?

3)	Is the area normally subject to land slippage?

4)	Will there be a need for the removal of large trees?

5)	Will construction endanger public utility services?

6)	Are water, sewage disposal facilities and electricity available on site?

7)	Is easy access to site available?

8)	Is there evidence of termite infestation in soil or trees?

9)	Determine the depth of foundation stratum.

10)	Determine the height of the water table if appropriate.

11)	Determine whether the soil is suitable for the construction of a soak-away pit.

12)	Select suitable areas for stockpiling aggregate.

13)	Select area for location of concrete mixer.

- Select location of materials storage shed.

15)	Select areas for stockpiling top- soil and excess fill of applicable

16)	Select area for erecting workbench

17)	Has planning permission been obtained?

18)	Do you have a surveyor's drawing of the site?

19)	Have you found the location of all boundary markers?

Having completed the above checklist, possible construction problems will have been highlighted and requirements of plant and materials made known.  It is recommended that an engineer or other appropriate professional be consulted when foundation problems are evident.

1.3	Site Clearance

a)	Care should be taken to preserve any trees on the site.  If trees have to be removed, special care shall be taken to remove totally, all roots and stumps of felled trees.  Such vegetable matter shall be removed from the site.

The Director may require an inspection of the site before the site is cleared of trees and other growth.

b)	The area where the building will be situated should then be stripped of top soil and this material stock piled in a suitable area for later use during landscaping.

1.4	Material Storage

a)	Areas should be allocated on the cleared site for the storage of materials.  Coarse and fine aggregate for the mixing of concrete and mortar should be placed in separate heaps in a location near to the concrete mixer and/or concrete mixing area.

b)	Cement, nails and finished materials requiring protected storage should be stored in a shed which is weather tight and has a wooden floor raised not less than four inches off the ground.

c)	Reinforcement should be stacked off the ground to reduce corrosion.

1.5	Batter Boards

a)	The building should be properly set out.  Batter boards, which are horizontal boards parallel to the sides of the building and supported by vertical boards driven into the ground may be erected in convenient locations near the four corners of the building, and to these boards should be transferred the building lines and levels for the project.

b)	The floor level is usually marked on the batter boards and used as a permanent reference.  All wall lines and levels shall be referred to these boards.  Periodic checks shall be made to see that these boards have not been shifted from their intended positions.

1.6	Roads and Pavings

a)	The roads and pavings dealt with in this Section are those suitable for  use as driveways and parking areas for private cars and light goods vehicles only.  Roadways should not be less than eight (8) feet wide and must conform to the relevant Planning Regulation.

b)	The choice of flexible (asphalt) or rigid (concrete) paving is largely influenced by the soil conditions at the site and the cost of driveway.  Gravel roads and pavings are acceptable if adequate drainage is available and if the gravel or crushed rock is reasonably hard, free from clay, and would not be easily crushed by the light traffic.  Adequate provision for drainage should be made.

c)	Where firm soils or rock is present, any paving type may be used.  Where soft soils are present a flexible paving or gravel is recommended.

d)	For all kinds of paving the top soil should be removed and replaced by a minimum of 6" of compacted, granular material.

e)	For rigid pavings, a concrete slab with a minimum thickness of 4" is added, reinforced by A142 welded wire mesh or equivalent, placed 1" below the slab surface.  For flexible pavings a minimum thickness of 2" of asphalt cold mix or hot mix is applied and compacted by roller.

f)	Single surface dressing is acceptable for light use driveways, or alternatively a sand seal applied on crushed limestone.  The Chief Technical Officer, Ministry of Communications and Works should be consulted when any questions arise.

2.	EARTH MOVING

2.1	Excavations

a)	Excavations for foundations shall be carried out along the building lines to the depth of the foundation stratum identified as suitable.  (See "1.2 - Checklist for Site Conditions", Items 9 and 10). It may be desirable to seek the assistance from the Physical Planning Division or an experienced engineer in determining the appropriate level for the foundation.

b)	Excavations not exceeding five (5) feet in depth may generally be without planking and strutting, which is a system of braced timber walls erected against the faces of the excavation to prevent collapse. For excavations exceeding five (5) feet the extent of planking and strutting necessary shall be determined by the nature of the soil and the location of the water table.

c)	Where collapse of the side of excavation is anticipated, all excavation in excess of five feet depth shall be planked and strutted.

d)	Where the foundation is in rock, it is necessary to excavate at least 2" to 4" so as to provide a key for the foundations.

e)	The bottom of all excavations shall be level and firm.  Where loose materials are encountered, foundation bottoms shall be compacted by ramming.

f)	Where excavations have been carried beyond their generally required depth, either by accident or design, the deep areas shall be backfilled with compacted, approved material or with concrete of proportions 1:3:6 by volume.

2.2	Backfilling

a)	Backfilling around foundation walls and under floor slabs shall be carried out using only suitable, selected materials. Unless the floor slab is reinforced to act as a suspended slab, the depth of fill should not exceed 3 feet.

b)	Suitable fill material may be brought to the site or obtained from excavated material, provided always that such material is free of substantial amounts of clay or organic matter.

c)	All backfill shall be well compacted in layers not exceeding 6" in thickness where compacted is by hand or small mechanical compactors.  Where vibratory rollers are used, the thickness of layers may be increased to 9 inches.

d)	Where backfilling under floor slabs has been effected using hard core, a 2" layer of sand shall be applied to the top of the compacted hard core to protect damp proof membranes from puncture.

e)	A damp proof course of polythene sheeting is sometimes applied between the top of the fill and the bottom of the floor slab to prevent damp from travelling upwards from the foundation to  the wall, especially in areas of high water table. However polythene sheeting if punctured during construction will not be an effective damp proof course, care should therefore be taken to prevent puncture when placing polythene.

f)	Other preferred methods of providing a damp proof course may be used, such as a dense concrete layer or bitumen on the foundation courses. Any such measure should be discussed with the Building Inspector, and should be a proven method of preventing the upward movement of moisture.

2.3	Compaction

Compaction of some soils is necessary to increase the bearing value of the soil. However the characteristics of the soils may best be known through local experience of the behaviour of the soils. For example, the compaction of organic clays is poor to very poor, while the compaction of sandy soils or sand clay mixtures by a rubber tired roller should provide good results.

It is recommended that where clay is encountered, professional advice be sought before planning the foundation.

3.	EARTHQUAKE AND HURRICANE CONSIDERATIONS

3.1	Earthquake Resistant Construction

3.1.1	General

a)	It is widely recognised in the building industry, that many of the Caribbean Islands are in an earthquake zone and have suffered varying degrees of damage due to earthquakes.

b)	Because of this situation, buildings must be designed and constructed so that they have some resistance to the shaking or lateral forces produced by earthquakes.

3.1.2	Effect of Soil Type

a)	The type of soil at the site may have a significant effect upon the resistance of the building to an earthquake. However for buildings within the scope of these Guidelines the effect of the soil type is not so significant provided that the building is not constructed on loose saturated sands, which may liquefy during an earthquake and cause collapse of the building.

b)	The earthquake may also, due to shaking of the ground, compact loose sands or fill material, and if a building is constructed on such material, the building will be damaged.

3.1.3	Effect of High Seas

Buildings on coastal areas may suffer due to high waves produced by earthquakes, and therefore the siting of the building in relation to the sea level is very important.

3.1.4	Building Shape

a)	The success with which a building survives an earthquake is greatly affected by its shape in plan, the way the building is tied together and the quality of construction.

b)	Most buildings with a simple rectangular shape with no projections (or only short projections) perform well under earthquake conditions provided the construction is adequate.

c)	Long narrow buildings should be avoided by limiting the length to three times the width. If the building must be longer, then it should be divided into separate blocks with adequate separation. Figure C-1 illustrates desirable and undesirable plan shapes.

d)	Rectangular buildings with well inter-connected cross walls are inherently strong and therefore desirable.

3.1.5	Appendages

Where buildings have decorative or functional additions or appendages such as window hoods, parapets and wall panels etc. extreme care must be taken to ensure that they are securely fixed, since many of such items tend to fall easily and may cause damage during an earthquake.

3.2	Rules for the Construction of Earthquake Resistant Buildings

It is recommended that the following rules be followed for the construction of buildings in earthquake prone areas:

3.2.1	Location of Openings

a)	The location and size of openings in walls have a significant effect upon the strength of a wall and its ability to resist earthquake forces.

b)	Openings are to be located away from a corner by a clear distance to at least 1/4 of the height of the opening. It is recommended that the minimum distance be 15".

c)	The total length of the openings should not exceed 1/2 the length of the wall between consecutive cross walls. (See figure C-2).

d)	The horizontal distance between two openings should not be less than 1/2 the height of the shorter opening. (See fig. C-2)

For two storey buildings, the vertical distance from an opening to one directly above it should not be less than 2'-0", nor less than one half the width of the smaller opening.

3.2.2	Masonry Buildings

An important factor contributing to the earthquake resistance of concrete masonry buildings is the detailing and placing of steel reinforcement. The design of a reinforced concrete frame building should be undertaken by experienced engineers. The reinforcing guide given in this section therefore must only be used for simple single storey buildings constructed of good quality concrete blocks.

The minimum quantities of reinforcing to be used have been set out in Section D Subsection 3.4. The following is a checklist which should be used to ensure that all necessary steps have been taken:

a)	Vertical reinforced columns should be placed in all masonry walls with a maximum dimension between columns of 16 ft.

b)	All exterior walls should be reinforced as shown in 3.4 of Section D. Interior walls also require reinforcement for earthquakes.

c)	All vertical wall reinforcement must be securely fixed to the foundation and belt course.

d)	Where vertical reinforcement is not a continuous bar, adequate lap lengths are to be provided and securely tied.  (See Table D-1).  Minimum lap length for a 3/8" bar should be 1'-6".

e)	Reinforced block work cores are to be filled after the laying of every three courses with well compacted concrete, or poured grout.

f)	In addition to the minimum vertical wall reinforcement, all wall, corners and junctions are to be reinforced.  (Figs C-3 and C-4).

g)	All vertical reinforcement is to be securely fixed to the ring beam. Horizontal reinforcement must be placed every 3 courses.

h)	Window and door jambs should be reinforced and tied into the lintels.

i)	Gable walls are to be reinforced by bars fixed to the concrete beam at eaves height and to the sloping ring beam at the top of the gable.  (Fig. C-5).

3.2.3	Timber Buildings

Although the foregoing principles apply to concrete block buildings, there are two additional areas of concern with respect to timber buildings:

a)	All corners and intersections must be adequately braced.

b)	Earthquake forces tend to remove timber buildings from their supports by shaking. Because  of this it is important to securely fasten sills to foundations.

3.2.4	Steel Buildings

a)	While steel buildings are generally beyond the scope of these Guidelines, it should be noted that the natural ductility of steel protects the frame itself from severe damage.

b)	However, in many cases concrete block walls are used and the precautions already listed for these walls will apply.  The wall reinforcement must now be anchored by welding to the steel columns and beams, or the steel frame encased in concrete in which case the wall reinforcement can be tied into the concrete cage encasing the steel frame.

3.3	Hurricane Resistant Construction

3.3.1	General

a)	It is very important in the Caribbean to be ever conscious of the fact that the region lies in the hurricane belt.

b)	Because of this, hurricane resistant construction principles must be adhered to if safe buildings are to be erected.  This section gives general principles for safe hurricane resistant design, and it is recommended that the details shown in these guidelines must be adhered to in order to ensure safe construction.

c)	For the buildings within the scope of this document the areas most vulnerable to hurricane forces are the roofs, windows and walls.

d)	The underlying objective of hurricane resistant construction is to produce a building that will not collapse during a hurricane. The building must be standing and its occupants should be safe.

3.4	Rules for Construction of Hurricane Resistant Buildings

To achieve safe construction, certain basic safeguards are necessary:

3.4.1	Building Site.

a)	Buildings sited in exposed areas (e.g. on the brow of a hill) are most vulnerable, while those sheltered by natural topography are less vulnerable.  Buildings sited in gullies or in river beds are very vulnerable as they are subject to severe damage by floods caused by the heavy rains which often accompany a hurricane.

b)	In siting the building, therefore, steep slopes and edge of cliffs should be avoided, as well as other conditions such as steep sided valleys where exceptionally high wind speeds are found.

c)	Where building on a steep site is unavoidable, reinforced concrete tie beams should be constructed to reduce the untied height of the columns to a maximum of 10 feet as shown on Fig. C-7. It is advisable to seek professional assistance for such construction.

3.4.2	Roofs.

a)	Experience and research have shown that flat roofs are vulnerable to high winds.  In an effort to reduce the uplifting wind forces on the roof, the roof pitch should be not less than 25 to 30 degrees.  Hip roofs should be used, since this shape of roof has been found to be more hurricane resistant than the gable roof. (Fig C-7). Where gable roofs are used they should be strongly constructed as shown in Figure C-8.

b)	Attention should be given to the location of fixings used for the roof cladding.  It is necessary to provide additional fixings at the roof edges and ridge, since high localised pressures are produced in these locations.

c)	The high suction along the eaves can be reduced by the use of a parapet, but then this has to be adequately strengthened, so that it does not become a hazard during a hurricane or in the event of an earthquake.  Parapets were very popular in some Caribbean Islands because of the protection offered to roof eaves.

d)	Roof overhangs also experience high local pressures and, where possible, these should be kept to a minimum or removed completely.

e)	Where buildings have roofed patios or verandas, their roofs should be separate structures rather than extensions of the main building roof.  A patio or verandah roof may be lost without endangering the safety of the main roof.

f)	The main roof must be securely fixed to the ring beam and ridge beams and details for achieving this are shown in (Figs C-9, and C-10).

3.4.3	Windows and Doors.

Special attention must be paid to the installation of doors and windows, since the loss of a door or window during a hurricane will greatly alter the internal pressure of the building, thus adversely affecting its safety.  For this reason, glazed windows and doors should be fitted with wooden shutters.

3.4.4	Walls.

Although it may not be common for the walls of concrete block buildings to be destroyed during a hurricane, many concrete block buildings were completely destroyed during the passage of hurricanes David in 1979 and Hugo in 1989. For this reason, it is important that the wall reinforcement be properly anchored at the foundation and the ring beam levels. (Fig D-6)

3.4.5	Timber Buildings.

a)	Because of the relatively light nature of a timber building, extra precautions must be taken concerning uplift.  The building must not be lifted off of it foundations.  Care must therefore be taken to ensure that the entire structure is securely fastened to the foundations.

b)	The spaces between the supporting columns or piers should be filled in to prevent increasing the uplift forces. (Fig. C-11)

c)	As far as timber walls are concerned, in addition to bracing corners in both directions, diagonal braces or steel straps must be installed at the level of the top plate to provide rigidity of the  corners at that level. (Figs. C-12 and C-13)

d)	In summary, timber buildings must be securely anchored to the ground and the building structure itself must be tied together with timber braces or metal straps.

3.4.6	Steel Buildings

Extensive damage to steel frames has occurred as a result to recent hurricanes in the Caribbean. The damage resulted from, in some cases, underdesigned sections and in most cases poor maintenance which led to significant reduction in the sizes of critical members and thence to failure.

The principles for the design and construction of hurricane resistant steel buildings are:

a)	Ensure that the manufacturer has been provided with the proper information regarding the design wind speeds so that the building frame design can be in accordance with the Code.

b)	Ensure that there are adequate numbers and sizes of foundation holding down bolts, and that they are all in place and properly fixed.

c)	Ensure that there is adequate lateral support provided by cross bracing or horizontal ties or by cast in place concrete or masonry walls.

d)	Where concrete walls or concrete masonry are used, the connections between the steel frames and the walls must be carefully designed and constructed as per Section F Subsection 3.

e)	Ensure that the manufacturer's recommendations with regards to the construction of the roof and roof covering are followed.

SECTION C - FIGURES

Notes:

1)	Drawings are schematic and not necessarily to scale.

2)	Where dimensions are metric, they can be converted to imperial dimensions as follows:

6 mm		=	1/4 inch

10 mm		=	3/8 "

12 mm		=	1/2 "

20 mm		=	3/4 "

25 mm		=	1"

3)	Roof slopes where not shown should be a minimum of  25 degrees.

4)	Specifications for reinforced concrete and for mortar are given in these Guidelines and in the Building Code.

5)	Where trade names such as "A142" mesh and "Dur a Wal" are used, other materials of similar (or better) specifications can be used.