Grenada Sustainable Energy Plan

Draft

August1st., 2002

Background	3

Energy Sector Baseline	4

Electricity Sector	4

Transport Sector	4

Sustainable Energy Plan: Energy Sector Scenarios	5

Fuel Consumption	5

CO2 Emissions	5

Electricity Losses	6

Export Income Spent on Fuel Imports	6

Vehicle Use	6

Sustainable Energy Plan: Required Actions for Achieving Targets	6

Assessment of Market Potential	6

Grid-Tied Renewable Energy Initiatives	8

Independent Renewable Energy Initiatives	10

Energy Efficiency Initiatives	11

Transportation Sector	14

Conclusion – Benefits to the Nation and the World	15

Tables	16

Figures	19

# Background

The independent state of Grenada consists of the islands of Grenada, Carriacou, and Petit Martinique. Located at 11o 58’ North and 61 o20’ West, it is the southernmost of the Windward Islands, lying between Trinidad and Tobago to the south and St. Vincent and the Grenadines to the North.

The population of Grenada is 89,227 (July 2001 est.) and the Gross Domestic Product is EC$689.87 million. Exports in 2000 were estimated at EC$168.21 million and imports at EC$587.25 million. Approximately one-half of all earning from domestic exports is used for the purchase of fossil fuel and energy imports constitute approximately 11% of total imports. This primarily is used to serve the electricity and transportation sectors.

Grenada currently has no over-arching policy, plan or strategy regarding energy use. One of the recommendations emanating from the First National Communication on Climate Change is the development of such a plan. It was determined by the Government of Grenada that this is a priority issue and, accordingly, the Government, together with input from key stakeholders from all sectors of society, has developed this Sustainable Energy Plan. The Sustainable Energy Plan lays out a strategy by which the energy sector in Grenada may become more economically and environmentally sustainable. As result of the crosscutting nature of energy, the plan is decidedly integrated in its approach, impacting all sectors (agriculture, industry, tourism, residential, etc.) and aspects of life (health, education, economy, etc.). In keeping with the priorities of the nation, the key goals of this strategy are increased economic development, poverty reduction, and improved environmental protection. Specific goals include:

- Ensure adequate, stable, reliable, and economical energy services to sustain economic development, while meeting current and projected demand.

- Enhance the security of energy supply and services for all sectors of the economy.

- Allow reasonable incomes for businesses engaged in the local energy sector, while attracting international investment where appropriate.

- Promote energy efficiency and conservation at all levels of the economy in order to achieve optimum economic use of renewable and non-renewable sources of energy.

- Protect the local and global environment by maximizing the use of renewable-energy and energy-efficiency alternatives.

- Determine the extent of Grenada’s potential hydrocarbon resources with a view towards developing these resources if found in commercial quantities. This development should be done in a sustainable manner, to benefit both the economy and environment of the nation.

Additionally, success factors for this plan, which its effectiveness is contingent upon include proper enforcement of regulations, monitoring, and evaluation of related actions.

# Energy Sector Baseline

This energy sector baseline has been established for purpose of comparison. It indicates energy sector statistics over the past several years. It is used to project for future growth and, accordingly, set the goals for the sustainable energy plan. This baseline was developed with data from Grenada Electricity Services Limited (GRENLEC), the Ministry of Finance, and the Ministry of Works, Communications, and Public Utilities.

The principal sub-sectors responsible for fossil fuel use within the energy sectors are electricity generation and transportation. Table 1 indicates the fuel consumption and the CO2 emissions by sector for 1994-1998 and based on this data Figure 1 illustrates the fuel consumption by sector for 1998 and Figure 2 illustrates the CO2 emissions for the same year. Table 2 indicates the energy/economic indicators for 1995-2000.

## Electricity Sector

Grenada has one national electricity company, GRENLEC, which supplies electricity throughout the State of Grenada. The government, WRB Enterprise (a US-based private company), employees, and smaller shareholders own GRENLEC. It supplies energy for over 90% of the population in Grenada and is the sole provider of electricity in Carriacou and Petit Martinique. Before being privately owned, ordinance No. 25 in 1960 granted GRENLEC the exclusive right to generate, transmit, distribute, and sell electricity in the country for a period of 80 years effect as from January 1, 1961.

Table 3 shows recent trends in electricity generation and sales by GRENLEC from 1997-2001 and based on this data Figure 3 illustrates the sale of electricity by sector for 2001. All current electricity generation by GRENLEC is diesel generation.

## Transport Sector

The transport sector is the second largest consumer of fossil fuel imports, following the electricity sector. Additionally, it is the source of a number of greenhouse gasses (GHGs), including principally, carbon dioxide (CO2), methane (CH4), nitrous oxide (N20). The energy demand in transport arises because of the need for motive force. Globally the transportation sector is estimated by the International Energy Agency (IEA) to have been responsible for about 25% of world primary energy use and 20-25% of global CO2 emissions in 1990.

The expanded market for imported, used vehicles, together with the relative ease or access to credit for motor vehicles from the established financial institutions have facilitated increased ownership of vehicles by more and more residents and citizens, and hence, constitute a major factor responsible for the recent increases in the vehicles fleet.

It is important to note here that while the importation of used (older) vehicles is price attractive and affordable to the majority of low and middle income individuals/families, these vehicles may:

- possess reduced fuel economy (miles per gallon), when compared to more recent models;

- possess no emission control devices or the devices may be dysfunctional, and

- the air-conditional units may still be reliant on the use of ozone deleting substances currently being phased out by the Montreal Protocol.

If this trend in vehicles is allowed to continue unchecked or in an uncontrolled manner it will contribute significantly to local and global environmental degradation and make it very difficult for Grenada to meet her international obligations under the Kyoto and Montreal Protocol.

The Marine Transport sub-sector consists of inter-island vessels/ferries, pleasure boats, cruise ships, water taxis, speed boats, aquatic sporting crafts, etc. This sub-sector, together with aviation in terms of the daily domestic flights between the islands of Grenada and Carriacou, are additional sources of fossil fuel consumption within the transport sector.

# Sustainable Energy Plan: Energy Sector Scenarios

The Sustainable Energy Plan develops “business as usual” and “best practices” scenarios for the energy sector for the following areas:

- Fuel consumption

- CO2 emissions

- Electricity losses

- Energy intensity

- Export income spent on imported fuel

- Vehicle Use

This plan strives to help Grenada to achieve the “best practices” scenario through the identification and eventual implementation of action items. This scenario is consistent with the objective of ensuring that sufficient, cost-effective, and reliable energy will be available to all customers in keeping with expected annual economic growth rate. The scenarios are based on data provided by the Central Statistics Office of the Ministry of Finance and GRENLEC and analysis undertaken by the Ministry of Works, Communications, and Public Utilities.

## Fuel Consumption

The fuel consumption “business as usual” projection predicts a continued rate of growth in fuel use of approximately 40 percent per five-year period, over three five-year periods of time (2000-2005, 2005-2010, 2010-2015). Although the “best practices” projection assumes continued growth in fuel consumption based on economic development and population growth, it projects 20 percent growth per five-year period (See Table 1 and Figure 4).

## CO2 Emissions

The CO2 emission is based on the same data set as fuel consumption. “Business as usual” projects a 43, 46, and 50 percent increase in emissions over the periods 2000-2005, 2005-2010, and 2010-2015, respectively. Given the lower rate of fuel consumption, the “best practices” projection also assumes a lower level of efficiency, aiming for 22 percent increases per five-year period (See Table 1 and Figure 5).

## Electricity Losses

Losses in the electricity sector since 1994 have ranged between 10-13 percent of total generation (See Table 3). Considering the small size of the system (transmission and distribution lines are not very long), this level is considered excessively high. A “best practices” scenario would reflect a high level of efficiency and a significant reduction in losses, and an average loss level of 4% (See Table 3 and Figure 7).

## Export Income Spent on Fuel Imports

Over the past five years, Grenada has spent an average of 48% of its income generated from exports on imported fuel consumption. In 1998-2000, this amount was reduced significantly to 29, 24, and 27 percent. “Business as usual” projects that the expenditure will… XXXXX. The “best practices” scenario assumes a significant reduction in fossil fuel imports in comparison to revenue generated by export down to XXXXX. (See Table 2)

(NEED TO ASK STATISTICS OFFICE TO DO A PROJECTION FOR FUEL PURCHASES AS PORTION OF EXPORT REVENUE)

## Vehicle Use

Currently there are 22,523 registered vehicles in Grenada. Given XXXXX, for the purpose of this analysis, the Ministry of Works, Communications, and Public Utilities has estimated that 25% percent of the registered vehicles are no longer in use, the number of vehicles on the road is closer to 16,892. Given historical trends, this amount is expected to increase at a rate of 10% annually, bringing that amount closer to 50,000 in 2010 (see Figure XXXX). [this amounts to ~ one vehicle per adult; is such a trend likely to continue for this period? A more reliable model may be a logistic curve, reflecting an upper bound due to the size of the population. That is, there is need to model the market potential based on a wave of adoptions, which will lead to slowed growth as the market saturates and as replacement buying begins to dominate.]

# Sustainable Energy Plan: Required Actions for Achieving Targets

Attaining the foregoing targets is considered feasible, according to the information obtained by the Government of Grenada for the development of this Sustainable Energy Plan. However, the present policy and regulatory framework requires adaptation to put in place suitable market rules and signals. Recommended actions for creating such an environment in which sustainable energy actions are implemented are described below.

## Assessment of Market Potential

- Project Identification for Renewable Energy: To expand the use of renewable-energy and energy-efficiency measures, it will be critical to ascertain, at least in broad terms, where key project opportunities exist. In the case of renewable energy projects, one of the main prerequisites to attracting potential investors is a basic set of data identifying key resource locations and describing the likely quantity and quality of such resources. A broad assessment of renewable energy resources, including wind, biomass, solar, and geothermal, will be assembled. In some cases, such as wind energy, resource assessments have been undertaken for several years but have not resulted in the commercial development of a project. As part of this Sustainable Energy Plan, additional site-specific assessments will continue in anticipation of locating quality resources with the potential for commercial exploitation. All project identified must integrate the issue of natural disaster preparedness into their design and execution.

Action:		Conduct renewable-energy resources assessments for wind, biomass, geothermal, hydro, and solar energy and compile these into a single Renewable Energy Resource Database for use in promoting Grenada as a possible destination for renewable-energy investments. Include past studies and assessments in the database.

- Project Identification for Energy Efficiency: Similar analyses of technical potential are also required for the energy-efficiency market. It is assumed that opportunities for electricity savings, through conservation, the use of high efficiency technologies, and better management of demand, are plentiful.

Action: 	An analysis/survey of the market potential for energy efficiency measures will also be undertaken. This will review generation and consumption patterns throughout the country and in each of the key sectors. These analyses will be used in the design of appropriate energy-efficiency measures and in efforts to attract entrepreneurial initiatives focused on energy savings.

- Petroleum Exploration: Given Grenada’s geographical location, close to the oil and natural gas fields currently being exploited by Venezuela and Trinidad and Tobago, there is a potential that the nation also possesses hydrocarbon reserves.

Action: 	Resolve the maritime boundary issues with Trinidad and Tobago and Venezuela.

Action: 	In collaboration with regional and international entities, explore the hydrocarbon reserves that lie within Grenada’s boundaries and extract those resources in a sustainable manner.

Action:	Through the execution of activities listed in this plan, ensure that Grenada is able to sell its hydrocarbon resources rather than use it itself, thus enabling the nation to use its revenue for development purposes (e.g. education, health, environment). This will serve to benefit the economy and environment of Grenada.

## Grid-Tied Renewable Energy Initiatives

- Electricity Generation Mix: The current generation profile and immediate plans for expansion consist entirely of diesel-powered generators. As Grenada has not yet exploited its potential fossil fuel resources, this dependency on imported petroleum products makes the nation extremely susceptible to swings in the international oil market. The use of diesel projects damages the environment (loss of biodiversity, increased greenhouse gases, etc.). Development of micro-hydro, solar, wind, biomass, and geothermal projects would result in a positive impact on the national economy and the environment and ensure domestic energy security.

Utilities often make choices about adding capacity according to their historical practices and often utilities tend to be conservative, reluctant to try new technologies. In the case of Grenada, the utility has indicated an interest in using renewables-based generation, yet has been unable to do so to date. Since the country has determined that it is in its best interest to add more renewable energy to its generation portfolio, government will mandate the addition of such capacity in the national system. Such a strategy may dictate that the utility must have available (or must deliver) a specific percentage of its electricity capacity via renewable-energy systems. The mandated portion may increase over time to let the utility gain experience with them gradually.

Action:	Based on achieving “best practices scenarios”, establish a Renewable Energy Portfolio Standard (RPS) for Grenada.

Grenada is blessed with abundant wind resources that can provide substantial generating capacity into the grid on both the island of Grenada and Carriacou.

Action:	Review wind assessment reports to date and conduct up-to-date analysis. Based on the assessment develop a pilot wind projects that connect to the grid on the island of Carriacou and on the east coast of Grenada. Utilize wind turbines, when appropriate, for remote applications.

- Adopt Policies that Encourage Private Power Development: Achieving specific renewable-energy targets in Grenada might not occur under the current exclusive arrangements currently afforded to GRENLEC. In many cases independent power producers with experience in renewable energy might be able to better develop these projects. Therefore, policies and regulations that permit and encourage Independent Power Producers (IPPs) will be developed. These regulations will describe the potential relationships between the IPP and the utility which might purchase arrangements, under which the third-party power developer sold electricity to the utility or, alternatively, wheeled power on the utility lines to specific consumers (such as a hotel). The Government is currently considering an Electricity Supply Act, which would adopt this policy.

Action:	The Government of Grenada will continue to explore alternatives to the current electricity monopoly granted to GRENLEC, including consideration of provisions that make possible independent power projects that sell electricity to the utility.

- Renewable-Energy Capacity and Awareness-Building Initiatives: Among the greatest impediments to the widespread use of renewable-energy technologies is the limited capacity of key decision makers and technicians. In addition, utility officials and engineers lack the information necessary to select, develop, and use renewables within their system. Likewise, gaining technical capacity in the operation and maintenance of renewable technologies, would make it much more likely that the systems installed would be successful and achieved their full potential.

Action:	Establish a comprehensive renewable energy training initiative with the purpose of increasing the capacity to develop and utilize these systems among the utility staff and potential project developers. This effort will be conducted in cooperation with CARILEC. The Government will request financial and technical assistance for it, from such sources as the Caribbean Renewable Energy Development Project (CREDP), in which it is a participating country.

The Government has determined that it is in the country’s best interest to catalyse the use of sustainable energy technologies, including renewables. The policies and regulations resulting from this Plan will contribute to the accomplishment of this objective; however, the long-term success of such efforts depends on a high level of support from the general public. A well-designed awareness and promotion campaign would result in a population that not only was more receptive to assuming the initial risks of these alternatives, but also would demand the incorporation of cleaner energy systems.

Action:	Initiate a national renewable energy education and awareness programme aimed at all sectors of civil society, to communicate the overall goals of the government with respect to the country’s economic development, protection of the environment, and the advantages of renewables. This programme would include an extra-curricular component, including activity development to introduce energy issues to youth.

- Establish Renewable-Energy Feasibility and Project Investment Fund: It is recognized that the initial cost of investment in renewable-energy facilities, coupled with the perceived risks of their use, may make it difficult for project developers to attract financing. This situation often presents itself in the preparatory phases of the potential project (i.e., pre-feasibility and feasibility studies), but may also include project financing for well-designed, commercially viable projects. Thus, it is critical to make funds available for investment in sound renewable-energy project opportunities. While such funds would not account for the entire investment of any project, they would be targeted at catalysing additional resources and serving as seed capital for worthy ventures.

Action:	The Government of Grenada will take the lead role in the creation of a dedicated renewable energy fund. This fund will provide concessional financing for renewable energy project feasibility studies and for project investment. The Government of Grenada will seek funds from several institutions, including the CREDP, the World Bank’s Prototype Carbon Fund, and international investors and donors to catalyse this financing.

- Establish Policies to Encourage and Enable Auto-Generation and Co-Generation: At present, GRENLEC is the only electricity provider permitted within the national grid framework. If a commercial or industrial property were to generate its own electricity, the utility would not continue to provide it with grid-based electricity. Thus, it is all or nothing for the potential auto-supplier. This discourages would-be entrepreneurs, such as a hotel that would otherwise choose to install renewable energy generation on its property but would still need the grid to meet part of its demand on occasion or at all times.

Action:	The Government of Grenada will consider the establishment of policies that permit companies to generate their own electricity while still maintaining continuous link to the power grid. This policy may also include a mechanism that permits auto-generators to sell excess capacity back to the utility.

- Establish Comprehensive Renewable Energy Regulations: In the absence of comprehensive power-sector reform, specific regulations will be required to govern areas such as independent power generation and the pricing and use of renewable-energy technologies. Also, an independent regulator with enforcement powers will be required for their implementation.

Action:	Electricity regulations and an independent regulator governing the generation and use of private power, and specifically renewable-energy technologies, will be established.

- Land Ownership Concerns: GRENLEC and the Government of Grenada have identified several key sites for development of wind energy. This land is privately owned and negotiations with the landowners to date have not been fruitful. As a result, GRENLEC is unable to proceed with its intention to develop renewable energy generation capabilities.

Action:	The Government will play a lead role in facilitating access to prime sites for renewable energy development through negotiation with the current owners of the land.

## Independent Renewable Energy Initiatives

- Create a National Solar Water Heating Initiative: The abundant direct solar radiation in Grenada offers tremendous potential for solar-based water-heating applications. It is estimated that the sun provides about 6 KWh of energy every day for every square meter of land. Solar water heaters have proved technically viable and economically efficient in other Caribbean island nations. Given the high cost of electricity, and the significant load required for water heating, which show the cost-effectiveness and relatively quick return on investment required for solar water heating systems, the potential for a national awareness and promotion initiative is considerable.

Action:	Establish a national solar water heating awareness initiative to target both the residential and the commercial sectors. This initiative will be linked to the energy efficiency activities described below.

Action:	Establish financial incentives for the purchase of solar water heaters (e.g. tax rebates).

Action:	Provide assistance to manufacturers and vendors of solar water heaters to enter the market of Grenada.

- Assess the Potential of Widespread Solar Photovoltaic Systems Installations: Solar Photovoltaic (PV) systems offer advantages for a diverse set of applications. With its price falling and the cost of traditional electricity in Grenada remaining high, increasing PV may be a cost-effective alternative in several areas. The assessment of PV in various applications will serve as a first step to eventual widespread use of PV systems.

Action:	Assess the potential and cost-effectiveness of solar PV systems for a variety of applications, both connected and unconnected to the grid. Such applications may include back-up power for hurricane shelters and schools, demonstration units at gasoline service stations, and demonstration units at government buildings.

- Analyse Biomass Use and Opportunities: Fuelwood is currently being used on a small-scale for cooking and charcoal production. Agricultural waste is not efficiently being used for energy production (with the exception of distilleries). There is no up-to-date analysis of this use pattern and its impact on human health, the environment, and the economy, as the last study was conducted 20 years ago.

Action: 	Conduct an analysis on biomass use. Determine its impacts on human health, the environment, and the economy. In particular, examine the issues of deforestation and health threats resulting from the use of fuelwood for cooking and charcoal production. Provide recommendations for energy patterns based, in part, on the analysis (e.g. replacement of fuelwood with liquid petroleum gas, biogas, etc.).

Action:	 Encourage the use of appropriate biomass technologies in the agricultural industry and other industries in order to improve efficiencies. Examples include biogas-digesters.

## Energy Efficiency Initiatives

- Conduct a Comprehensive Energy End Use Analysis: As a means of determining the energy use patterns of the various sectors of the economy a comprehensive study will be undertaken. The results of this analysis will provide the necessary information regarding potential areas for energy efficiency applications and will serve to guide the other activities described in this Plan.

Action: 	A study of energy end use practices in all sectors (public, commercial, residential, etc.) of the economy will be conducted in collaboration with GRENLEC. This report will highlight key opportunities for energy savings.

- Initiate a Comprehensive Capacity-Building Initiative Among Personnel of Utilities Commercial Energy Plants, and Other Relevant Organizations: A critical first step toward the success of any energy-efficiency initiative involves the development of appropriate awareness and technical capacity among the organizations and individuals that will participate in these programs. Traditional utilities and other energy-sector personnel are often hesitant to promote strategies that effectively reduce the demand for electricity. However, it has been well demonstrated that there are many business opportunities for both the utility and third party organizations that may result in attractive investments. Further, the most cost-effective reductions in environmental impacts from the energy sector are typically derived from efficiency improvements. There are many areas of training that will be useful in laying the foundation for a solid energy-efficiency program in Grenada. Such measures should address wide-ranging areas as business development and creative strategies to technical aspects of energy efficiency. This initiative will seek to build a consensus among both existing electricity-sector personnel and potential entrepreneurs, that promoting greater energy efficiency in Grenada represents a win-win opportunity for all.

Action:	 Implement a comprehensive energy-efficiency training program for utility personnel, hotel developers and engineers, potential entrepreneurs, and other relevant persons.

- Support and Assist in the Establishment of Energy Service Companies (ESCOs): ESCOs are businesses that derive their income by generating energy savings for their clients. They may be affiliated with the utility or operate as independent, third party enterprises. They typically provide services for commercial businesses, such as the hotel industry. It is common for them to enter into a contract with a client, such as a hotel, whereby they identify and help to implement opportunities to generate energy savings by retrofitting energy-consuming technologies and changing patterns of electricity consumption. The ESCO receives payment for the consulting and engineering services it provides, which are typically less than the overall savings accruing to the client. Thus, it is a win-win relationship for both. Significant energy savings might be realized via the government sector, industry, tourism and other commercial buildings.

Action:	Catalyse the creation of one or more ESCOs in Grenada. The Government, in cooperation with an energy-sector venture capital fund (E&Co.), will assess the potential and seek opportunities to invest in one or more entrepreneurial enterprises.

- Launch a National Demand-Side Management (DSM) Initiative Designed to Reduce Residential Energy Consumption: Residential energy consumption patterns in Grenada offer the potential for DSM measures to reduce electricity demand. Effective DSM programs require well-designed and targeted campaigns that communicate to the population the need for and potential benefits from reducing consumption. They often include printed materials describing successful applications of energy efficiency measures and the economic savings realized (newspaper advertisements or articles, brochures, and utility-sponsored seminars).

Action:	Implement a residential demand-side management (DSM) program intended to reduce consumption in the residential sector by 10% by the year 2010.

Action:	Establish regulations setting energy-efficiency standards for new construction.

Action:	Review, and possibly modify the existing rate structure for electricity consumers to encourage more rational use of electricity (e.g. higher rates for peak hours).

Action: 	Establish a public awareness campaign through both the media and educational institutions regarding energy conservation. This would include curriculum development to introduce energy conservationfrom pre-primary through secondary schools in science and social studies classes.

- Support the Establishment of and Participate in the Caribbean Energy Efficiency Development Project (UNDP/GEF, PDF Block B): A regional project for the promotion of energy efficiency is being prepared for implementation by the United Nations Development Programme (UNDP) with funding from the Global Environment Facility (GEF). This project will assist participating countries in the identification and execution of energy-efficiency programs. This project has the potential to offer Grenada technical and financial resources for its energy-efficiency activities.

Action:	Participate as an active member country of the newly launched Caribbean Energy Efficiency Development Project.

- Establish Guidelines for Energy Efficient Practices in all Government Buildings: By establishing standards for energy efficient practices in all government buildings, two important objectives may be achieved. First, given the considerable number of government installations, reducing energy consumption in this sector will contribute to the national efficiency goals and reduce costs.  Secondly, the Government will serve as an example to other sectors in the economy by adopting energy efficiency practices. The Government may implement a variety of energy efficiency practices, including the use of energy efficient lighting and other appliances, training and implementation of energy conservation practices, and design and acquisition of efficient buildings for all new locations.

Action:	Assess the potential for energy efficiency practices in all Government buildings. Based on this assessment, develop a standards manual for use by all government agencies describing recommended and/or required practices for existing and new buildings and equipment.

## Transportation Sector

- Establish Regulations Requiring, or Providing Incentives for, the Purchase of Higher Efficiency Vehicles: Vehicle fuel economy has a large impact on the volume of emissions. Newer, high efficiency vehicles emit lower emissions per kilometre/mile travelled. Increasing the number of high-efficiency vehicles in Grenada might be achieved by regulations requiring certain emissions/efficiency standards, by offering tax incentives for the purchase of efficient vehicles, or a combination of these.

Action: 	Analyse potential alternatives for improving the fuel efficiency and reducing the harmful emissions of Grenada’s vehicle fleet. This could include tariffs structures, fiscal incentives and disincentives, and public awareness.

- Improvements in Public Transportation: Public transportation plays a significant role in the larger transportation sector. Effective public transportation can significantly decrease the number of private vehicles on the road, thus lowering fossil fuel consumption, greenhouse has emissions, and traffic congestion.

Action: 	Promote improvements in the public transportation, including the route evaluation, increased use of efficient vehicles, designated parking areas and associated rates, and public awareness to bus owners and the general public.

Action:	Manage traffic patterns to reduce congestion and ease traffic flows entering and leaving the city zone.

Action:	Establish a public awareness campaign through the media, community groups, and educational institutions regarding public transportation and alternative forms of transportation/fuel (e.g. buses, scooters, hybrid vehicles, etc.).

- Transportation Information: Currently it is difficult to plan and manage within the transportation sector, due to the lack of available information. A complete database of vehicles in use, their engine size, and fuel use, and their age will help to better manage the sector and assist policy makers in their decision making process.

Action: Establish a comprehensive transportation sector database.

# Conclusion – Benefits to the Nation and the World

Given the fact that energy is an essential input in the production of all goods and services, and that currently Grenada relies almost exclusively on the importation of fossil fuels to satisfy her energy needs, Grenada’s Sustainable Energy Plan will contribute significant benefits to the national economy, protection the environment, and poverty reduction. The Plan will also allow Grenada to serve as a global model for the benefits of sustainable energy policies. These benefits are substantial, as the use of fossil fuel contributes to global climate change, local environmental damage, and growing debt due to significant foreign reserves being spent on fuel imports.

The success of this plan is heavily reliant on regulation, monitoring, evaluation and ownership by a broad spectrum of civil society.

# Annex

## Tables

| Table 1: Energy Sector Fuel Consumption (TJ) and CO2 Emissions (Gg) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Fuel Consumption (TJ) | 1994 | 1995 | 1996 | 1997 | 1998 |
| Energy Industries (GRENLEC) | 845.46 | 886.6 | 947 | 1025.95 | 1133.77 |
| Manufacturing Industries and Construction | 57.35 | 49.28 | 35.05 | 117.08 | 138.1 |
| Road Transport | 692.21 | 727.98 | 827.78 | 836.71 | 848.36 |
| Marine Transport | 53.26 | 74.98 | 110.5 | 91.89 | 137.42 |
| Commercial/Institutional | 175.34 | 201.21 | 209.28 | 120.48 | 122.47 |
| Residential | 153.83 | 266.65 | 116.21 | 157.34 | 160.03 |
| Agriculture/Forestry/Fishing | 5.58 | 3.7 | 4.84 | 5.04 | 3.22 |
| Total | 1983 | 2210.4 | 2250.7 | 2354.5 | 2543.4 |
| International Marine Bunkers | 5.92 | 8.33 | 12.22 | 4.94 | 15.23 |
| International Aviation Bunders | 132.02 | 290.3 | 313.74 | 205.73 | 459.22 |
| CO2 Emissions (Gg) |  |  |  |  |  |
| Energy Industries (GRENLEC) | 61.99 | 65.01 | 69.44 | 74.94 | 82.85 |
| Manufacturing Industries and Construction | 4.13 | 3.61 | 2.57 | 8.74 | 10.02 |
| Road Transport | 47.86 | 50.33 | 57.22 | 57.98 | 58.83 |
| Marine Transport | 3.81 | 5.38 | 7.93 | 6.54 | 4.53 |
| Commercial/Institutional | 12.2 | 14.31 | 14.94 | 8.55 | 8.63 |
| Residential | 9.76 | 16.76 | 7.5 | 9.94 | 10.1 |
| Agriculture/Forestry/Fishing | 0.4 | 0.27 | 0.35 | 0.37 | 0.24 |
| Total | 140.15 | 155.67 | 159.95 | 167.06 | 175.2 |
| International Marine Bunkers | 0.42 | 0.6 | 0.88 | 0.34 | 1.08 |
| International Aviation Bunkers | 9.32 | 20.52 | 4.43 | 14.52 | 32.48 |
| Source: XXX |  |  |  |  |  |

| Table 2: Energy/Economic Indicators |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 |
| Total Imports | 350405.5 | 416114.2 | 465753.5 | 550095.9 | 554791.3 | 664127.6 |
| Total Exports | 59138.7 | 55944.4 | 80390.9 | 112255.9 | 187591.9 | 210972.9 |
| Imports of Petroleum Gases ('000 KG) | 3757.4 | 3335.2 | 3932.5 | 3083.5 | 4086.4 | 4195.9 |
| Imports of Kerosene ('000 CU. M) | 0.2 | 10.2 | 7.5 | 11.2 | 8 | 0.3 |
| Imports of Gasoline ('000 CU. M) | 14 | 18.5 | 16.4 | 19.1 | 22 | 24.8 |
| Imports of Diesel ('000 CU. M) | 28 | 38.5 | 22.1 | 15.1 | 55.1 | 37.6 |
| Petroleum Imports (EC$'000) | 3981.2 | 4820.7 | 4766.7 | 4106.8 | 4789 | 7819 |
| Kerosene Imports (EC$'000) | 112.4 | 4607.4 | 3596 | 6031.1 | 4297.2 | 229.5 |
| Gasoline Imports (EC$'000) | 9125 | 12114.4 | 12490.8 | 14424.4 | 12115 | 20191.6 |
| Diesel Imports (EC$'000) | 11915.8 | 16952.4 | 13137.5 | 8444.6 | 24738.1 | 28433.9 |
| Total Hydrocarbon Imports | 25134.4 | 38494.9 | 33991 | 33006.9 | 45939.3 | 56674 |
| Hydrocarbon Imports as a % of Total Imports | 7.17% | 9.25% | 7.30% | 6.00% | 8.28% | 8.53% |
| Hydrocarbon Imports as a % of Total Exports | 42.50% | 68.81% | 42.28% | 29.40% | 24.49% | 26.86% |
| Source: Digest of Trade Statistics 2000, Central Statistics Office, Ministry of Finance |  |  |  |  |  |  |

| Table 3: Electricity Operating Statistics in Grenada, 1997-2001 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2001 | 2000 | 1999 | 1998 | 1997 |
| Sales of Electricity by Sector (kWhs ) |  |  |  |  |  |
| Domestic | 50,225,178 | 45,932,002 | 43,929,817 | 41,024,181 | 38,338,387 |
| Commercial | 66,560,369 | 60,911,455 | 55,142,953 | 51,446,799 | 45,693,445 |
| Industrial | 5,373,323 | 4,975,593 | 4,786,353 | 4,673,200 | 4,380,306 |
| Street Lighting | 1,759,528 | 1,528,644 | 1,350,801 | 1,247,518 | 1,230,009 |
| Total | 123,918,398 | 113,347,694 | 105,209,924 | 98,391,698 | 89,642,147 |
| Generation (kWhs) |  |  |  |  |  |
| Gross Generation | 146,352,753 | 133,609,611 | 122,154,267 | 113,038,838 | 102,877,595 |
| Own Use | 3,558,274 | 2,907,464 | 2,763,480 | 2,481,450 | 2,558,601 |
| Net Generation | 142,794,479 | 130, 702,147 | 119,390,787 | 110,557,338 | 199,318,994 |
| Number of Consumers at Year-End |  |  |  |  |  |
| Domestic | 29,903 | 28,727 | 26,988 | 26,434 | 25,619 |
| Commercial | 4,346 | 4,321 | 3,915 | 3,802 | 3,358 |
| Industrial | 39 | 41 | 41 | 43 | 46 |
| Average Annual Usage per Consumer Class (kWh) |  |  |  |  |  |
| Domestic | 1680 | 1599 | 1628 | 1552 | 1496 |
| Commercial | 15315 | 14097 | 14085 | 13,532 | 13,607 |
| Industrial | 137,778 | 121,356 | 116,740 | 108,679 | 95,225 |
| Source: Grenada Electricity Services, Ltd. |  |  |  |  |  |

| Table 4: Electricity Production and Sales Data (Past and Projected) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| Installed Capacity (MW | 25.1 | 25.1 | 25.1 | 29.45 | 27.23 | 26.99 | 43.11 | 42.45 |
| Maximum Demand (MW) | 12.5 | 14.5 | 15.4 | 17.47 | 18.54 | 19.3 | 20.4 | 22.7 |
| Gross Generation (KWhs) | 83,583,690 | 91,835,485 | 98,007,398 | 102,877,595 | 113,038,838 | 122,154,267 | 133,609,611 | 146,352,753 |
| Company's Own Use (MW) | 3,385,600 | 2,342,970 | 2,177,581 | 2558601 | 2481450 | 2,763,480 | 2,907,464 | 3,558,274 |
| Net Generation (MW) | 80,198,090 | 89,492,515 | 95,829,817 | 100,318,994 | 110,557,338 | 199,390,787 | 130,702,147 | 142,794,479 |
| LOSS (% of Net Generation) | 11.3 | 11.21 | 12.73 | 10.64 | 11 | 11.88 | 13.28 | 13.22 |
| Sales KWhs) |  |  |  |  |  |  |  |  |
| Domestic | 30,024,328 | 33,533,720 | 35,706,530 | 38,338,387 | 41,024,181 | 43,929,817 | 45,932,002 | 50,225,178 |
| Commercial | 34,909,699 | 40,006,765 | 42,497,772 | 45,693,445 | 51,446,799 | 55,142,953 | 60,911,455 | 66,560,369 |
| Industrial | 5,035,751 | 4,727,642 | 4,201,939 | 4,380,306 | 1,673,200 | 4,786,353 | 4,975,593 | 5,373,323 |
| Street Lighting | 1,163,538 | 1,200,551 | 1,227,291 | 1,230,009 | 1,247,518 | 1,350,801 | 1,528,644 | 1,759,528 |
| TOTAL SALES | 71,133,316 | 79,468,678 | 83,633,532 | 89,642,147 | 95,391,698 | 105,209,924 | 113,347,694 | 123,918,398 |
| Number of Customers |  |  |  |  |  |  |  |  |
| Domestic | 22,997 | 24,073 | 25,043 | 25,619 | 26,434 | 26,988 | 28,727 | 29,903 |
| Commercial | 2,503 | 2,621 | 2,761 | 3,358 | 3,802 | 3,915 | 4,321 | 4,346 |
| Industrial | 51 | 49 | 48 | 46 | 43 | 41 | 41 | 39 |
| TOTAL CONSUMERS | 25,551 | 26,743 | 27,852 | 29,023 | 30,279 | 30,944 | 33,089 | 34,288 |
| Fuel Used (IG) | 6018302 | 5028511 | 5414943 | 10042549 | 8,466,935 | 9,354,694 |  |  |
| Fuel Cost (EC$M) |  |  |  |  |  | 17.3 | 27.9 | 26.2 |
| CO2 Emissions (Gg) | 61.99 | 65.01 | 69.44 | 74.94 | 82.85 |  |  |  |
| Kg of CO2 Emissions/KWh | 0.7417 | 0.7079 | 0.7085 | 0.7284 | 0.7329 |  |  |  |
| Source: GRENLEC Annual Report 2001 and GRENLEC Monthly Reports with Analysis by the Ministry of Works, Communications, and Public Utilities. |  |  |  |  |  |  |  |  |

## Figures

Figure 7: Electricity Losses

Figure 8: Historical and Projected Registered Motor Vehicles (1991 - 2010) – Based on data from Inland Revenue Department, Ministry of Finance 2000-2010 Projections, Ministry of Works