APPENDIX B

# MAIN DIFFERENCES BETWEEN WIND AND EARTHQUAKES

| # ITEM CONSIDERED | WIND | EARTHQUAKES EFFECTS |
| --- | --- | --- |
| Source of loading | External forces due to wind pressures | Applied movements from ground vibration |
| Type and duration of loading | Wind storm of several hours’ duration; loads fluctuate, but predominantly in one direction | Transient cyclic loads of at most a few minutes duration; loads change direction repeatedly |
| Influence of local soil conditions on response | Unimportant. | Can be important |
| Predictability of loads | Usually good. | Poor |
| Main factors affecting building response | External shape and size of building; dynamic properties unimportant except for very slender buildings | Response governed by building dynamic properties: fundamental period, damping and mass |
| Normal design basis for maximum event | Elastic response require | Inelastic response permitted, but ductility must be provided; design is for a small fraction of the loads corresponding to elastic response |
| Design for non-structural elements | Loading confined to external cladding | Entire building contents shaken and must be designed appropriately |