SECTION F

STEEL CONSTRUCTION

Contents

1.	MANAGEMENT

1.1	Ordering and purchasing

1.2	Handling

2.	FOUNDATIONS

2.1	General

3.	WALLS

3.1	General

3.1.1	Concrete walls

3.1.2	Metal cladding

4.	PORTAL FRAMES

4.1	General

4.2	Erection

5.	PERFORMANCE SPECIFICATIONS FOR SMALL STEEL FRAMED BUILDINGS

5.1	Introduction

5.2	Outline specifications

5.3	Other requirements

5.4	General review by the Authority

Figure

F-1	Steel Column

SECTION F

STEEL CONSTRUCTION

1	MANAGEMENT

1.1 	Ordering and Purchasing

a)	In many territories of the Caribbean islands, there is no local company undertaking the design and fabrication of structural steelwork components.

b)	For this reason, all steel structures to be erected are imported in dismantled form and erected on site.

c)	Most suppliers of steel buildings require information setting out the plan dimensions of the proposed building, its height, type of framing and design parameters such as basic wind speed, and gravity loading.

d)	Using this information the supplier will design the structures and provide a quotation.

e)	On receipt of approval, the fabrication of components is put in hand and arrangements for payments finalised.

f)	Payment is usually by letter of credit but other methods of payment may be agreed.

g)	The design and construction of steel structures are generally outside the scope of these Guidelines. However, there is an increasing number of steel shells being imported and used for factories and small manufacturing enterprises. The information contained in these Guidelines is intended to assist the builder in understanding the basic considerations for ordering, storing, and erecting the structure.

h)	It is further recommended that, in all cases when contemplating the use of steel structures, experienced professional advice be sought.

1.2	Handling

a)	The performance of a steel framed building will depend to a large extend upon the way it is handled in the early stages.

b)	Preferably, steel work should be cleaned of rust by shot or sand blasting and then painted with 2 coats of red lead oxide metal primer.

c)	This treatment will ensure that it reaches site in very good condition.

d)	Before erection, the steel work should be primed with red lead oxide primer to ensure that all surfaces are primed before the final decoration is applied.

Note: 	The basic wind speed and other factors to be used in the design must be in accordance with the factors described in Caribbean Uniform Building Code.

2.	FOUNDATIONS

2.1 	General

a)	Foundations for steel framed buildings generally consist of a reinforced concrete pad located at a suitable depth on a bearing stratum.

b)	The pad supports a stub column which has the column holding down bolts cast into it (see Fig. 1).

c)	The height of the stub column is such that the column base is located at the desired elevation.

d)	Since steel framed buildings are comparatively light weight structures, the footing size is usually arrived at by determining the size of the adequate anchor to prevent uplift due to wind. The size of footing should therefore be provided by the designer or supplier, and the horizontal restraint needed at the column bases must also be shown on the design.

e)	Because of this, it is not possible to provide specific design guidelines, since the size of the footing is dependant upon the forces which would be generated by the wind.  Experienced engineers would be required to carry out a wind analysis, the result of which would permit design of the footing to be undertaken.

f) 	When hollow concrete blocks are used to construct the walls, strip footings between the column footings are used for their support.  These strip footings are as described in Section C.1.

g)	Fig. F-1 shows the typical arrangement for the installation of a steel frame. The design of the stub columns (where necessary), the design of the footings and the connections of the exterior walls to the steel columns must be carried out by an experienced engineer.

3	WALLS

3.1	General

For steel framed buildings, two systems are generally used for walls.  These are hollow concrete block walls or metal cladding.

3.1.1	Concrete Walls

a)	When concrete walls are used, the information given in Section D. is applicable.  There are some minor differences concerning the anchoring of the wall reinforcement.  The vertical steel is anchored to the footing in the normal manner and anchored at the top by welding to the longitudinal beam, or fixed to a concrete beam constructed on top of the walls.

b)	The horizontal reinforcement is welded to the web of the columns. If the columns are encased in concrete the reinforcement can be carried to the face of the steel column.

c)	Care must be taken to fix these walls to the steel frames so as to provide lateral continuity to the walls and to prevent the wall from collapsing either under the shaking from an earthquake or from the pressures due to hurricane winds.

d)	It is however, sometimes necessary to install a flexible joint between the block wall and the steel column where the walls have not been used to provide lateral stability. In this event the steel frame must be adequately braced to accommodate the lateral loads without collapse.

3.1.2	Metal Cladding

a)	Where metal cladding is used, Z-purlins are attached to the columns with suitable fixings.

b)	The vertical siding, as the sheeting is then called, is attached to the Z-purlins in the normal manner employed for roofs. Fixings made in the valleys rather than on the crowns would provide greater hurricane resistance. Care must be taken to prevent leaks.

4.	FRAMES

4.1	Frame Types

a)	The type of frame most commonly used in the region is the portal frame with pinned bases.  This kind of frame is commonly used in factory shells, warehouses and some commercial buildings.

b)	The frame consists of two columns and two rafters and all connections are usually bolted, although welded connections are sometimes used.

c)	The portal frames are linked by longitudinal beams at the top of the columns and purlins between the rafters.

d)	The first and last bays are usually fitted with diagonal bracing in the plane of the columns and in the plane of the roof. This diagonal bracing is very important to the stability of the frame and must not be left out unless other arrangements are made to provide resistance to lateral loads.

e)	Where the end bays must be left clear for doors, the diagonal bracing is replaced by a longitudinal frame sometimes referred to as "goal-post bracing", or diagonal bracing placed in other bays.

f)	The roof bracing is usually of tubular steel while the column bracing is similar or is made of adjustable steel rods acting in tension, in which case it must be cross bracing.

g)	The roof is usually clad with corrugated galvanised or plastic coated steel sheets or corrugated aluminium sheets.  These sheets are fixed to the purlins by self-tapping screws fitted with flexible washers driven through the crowns of the corrugation.  The spacing of these fixings is as described in Section C Sub-section 3.3. If special aligning guns are used, valley fixings would provide much greater hurricane resistance than crown fixings.

4.2	Erection

a)	The steel frames are erected on the prepared foundation which will have the holding down bolts projecting above the level of the stub columns.  These bolts are usually cast in PVC sleeves so that some adjustment is possible.

b)	Firstly, the columns are lifted and slipped over the bolts.  Nuts and washers are put on and partially tightened.

c)	This assembly is then lifted and the connections to the two columns made.

d)	This process is repeated with the second frame and the two are then joined by one purlin on each side of the crown.

e)	After all the frames have been erected, the columns are plumbed and the holding down bolts are grouted, the longitudinal beams added and all connections completed.

f)	The remaining purlins, the roof bracing and the diagonal column bracing are then installed.

g)	The roof sheeting and walls are then added to the completed frames.

5.	PERFORMANCE SPECIFICATIONS FOR SMALL STEEL FRAMED BUILDINGS

5.1	Introduction.

a)	It is recognised that there may be significant savings in labour in the construction of pre-engineered buildings, especially in view of the technology affecting the design of such buildings. The criteria to be considered in the appraisal of pre-engineered buildings are:

i)	Resistance to wind and earthquake forces.

ii)	Resistance to corrosion.

iii)	Compatibility with environmental and aesthetic standards.

b)	Other economic and social criteria may also be considered by the Authority, but the structural characteristics must be in accordance with the minimum requirements of the Building Code.

c)	The following specifications are for the construction of small single storey buildings of less than 2500 sq.ft. The design of larger buildings must be carried out by a rational method in accordance with a Code or Standard approved by the Authority.

5.2	Outline Specifications

a)	Single storey building with room sizes no greater than 160 square feet with concrete floor and standard foundations.

b)	Eave height a maximum of 10 feet.

c)	Roof pitch 25 degrees minimum. Hipped roofs preferable. If roof is composed of light steel joists, the design and fixing of those joists must be in accordance with Section 17 of the Code.

d)	Roof sheets to be a minimum of 22 gauge if of aluminum, or 24 gauge if of steel. Purlins to be spaced a maximum of 2 ft on centre and fastened with drive screws at every crown; provided that the crowns are not further than 12" apart.

e)	Roof and cladding material if of metal to be protected with approved non-corrosive covering or galvanising.

f)	Roof overhang a maximum of 12"

g)	Joints and connections to be capable of withstanding the horizontal and vertical forces.

h)	Wall panels or cladding of steel plate shall be a minimum of 16 gauge. Steel plate panels should be no greater than 4 ft by 10 ft and be supported firmly at sill and plate levels. If panels are built up to form a sandwich board, the panels should be tested for resistance to hurricane force loads at

the unrestrained spacing recommended by the manufacturer. The connections between the panels and the steel frame must be shown on the drawings. If the panels are of corrugated sheeting, the gauge of the sheeting must be at least the same gauge as specified for the roof sheets.

i)	Intermediate studs are not to exceed 2 ft on centre and horizontal connector pieces to form unrestrained panels of no more than 2 ft by 2 ft should be installed. The use and spacing of horizontal connector or bridging members is dependent on the strength of the infill panel.

j)	Sill beam and plate members to be American Standard Channel Sections (or equivalent) at least 3 ins wide and flange thickness of 0.27 inches. If other steel sections are being used, the developer must show that such sections are capable of withstanding the vertical uplift loads imposed without tearing of the metal.

k)	The sill member should be bolted to the concrete floor slab or concrete beam at least every 2 ft on centre. The bolts should penetrate the concrete at least 4 inches.

l)	Where cladding is of concrete block, the developer must show in the drawings the connection between the cladding and the steel frames.

m)	All end frames should be cross braced, except where the developer can demonstrate on the drawings that the concrete block cladding is suitably connected to the frames and can act as lateral support to the frames.

5.3	Other General Requirements

a)	Each building to be provided with WC and bath or shower of at least 35 sq.ft in area.

b)	There should be two exit doors, one of which should be capable of accommodating a wheel chair.

5.4	General Review by the Ministry of  Communications and Works

a)	Developers should show that their proposed building has been subjected to, or can resist hurricane forces, either by laboratory tests or by actual previous exposure to hurricanes.

b)	All proposed technology will be seriously considered and examined by the Ministry of Communications and Works or by a Special Inspector appointed by the Authority pursuant to Section 114 of the Grenada Building Code, to determine whether the building is acceptable and responsive to the conditions in the State.

c)	The Ministry or Special Inspector may recommend changes which are, in its opinion, needed to make the building resistant to the environmental hazards. Such changes may include placing of extra holding bolts, installation of cross bracing in end frames or extra fastenings for roof sheets.

d)	However, it is the responsibility of the developer to show by supporting documentation that the building proposed is structurally adequate to resist the environmental hazards of high winds and earthquakes.