SECTION J

PRECAUTIONS DURING BUILDING CONSTRUCTION

Contents

1		GENERAL

1.1		Scope

1.2		Erection of Scaffolds and Safeguards

1.3		Electricity Lines and other Hazards

1.4		Underground Utilities

1.5		Duty of Care

2		DEMOLITION

3		EXCAVATION

# 4		CONSTRUCTION FENCES

5		STORAGE OF MATERIAL

5.1		General

5.2		Hazardous Material

6		LADDERS

7		SCAFFOLDS

8		TEMPORARY LIGHT AND POWER

9		SANITATION

10		OPEN FIRES

11		SPECIAL HURRICANE PRECAUTIONS

SECTION J

PRECAUTIONS DURING BUILDING CONSTRUCTION

1.		GENERAL

1.1		Scope

a)	The provisions of this Section shall apply to all work in connection with the erection, alteration, repair, removal and demolition of buildings and structures. It is the duty of all builders to make every effort to provide a safe working environment for workers on building sites.

b)	The Director may issue an order to stop work if any provision of this Section is violated. Work will not be allowed to recommence until the owner has complied with the provisions of the Section to the satisfaction of the Director.

1.2		Erection of Scaffolds and Safeguards

a)	The construction, erection, alteration and removal of scaffolds and the application, installation and setting up of safeguards and equipment devices shall be done by skilled workmen under the supervision of a person qualified by experience and training for such work.

b)	The builder or owner shall take immediate steps to remedy a safeguard, device or piece of equipment that is unsafe.

c)	Scaffolds, ladders stairs, fuel gas tanks and other devices or equipment regulated by this section shall be maintained in a good, safe and usable condition as long as they are in use.

d)	No ladder, scaffold, railing or other devices or equipment required or regulated by this section, or any part thereof, shall be removed, altered or weakened when required by the work.

e)	Scaffolds, temporary floors, ramps, stairway landings, stair treads, and all other walkway surfaces shall be kept free from protruding nails and splinters.  They shall be kept free from necessary obstructions so that the workers may move about safely.

f)	Protruding nails and tie wire ends shall be removed, hammered in or bent in a safe condition.

1.3		Electricity Lines and other Hazards

a)	Electric lines, moving ropes and cable gears, or similar hazards with which a worker might come in contact, shall be encased or guarded.

b)	No person, firm or corporation, either personally or through an employee or agent of another, shall operate or move any machine, equipment, material, scaffolds or assembly closer than six feet to any energised high-voltage overhead electrical facilities except with the approval of the Electrical Inspector or the utility company concerned.

1.4		Underground Utilities

Prior to making an excavation, drilling or otherwise disturbing the ground, the person undertaking the work, or causing such work to be done, shall contact all public utility organisations to determine the possible location of underground facilities to avoid the disruption of such facilities.

1.5		Duty of Care

It is the responsibility of the builder to ensure that all operatives in his/her employ are appropriately trained and that they can operate the equipment safely. It is also his/her responsibility to ensure that no member of the public is exposed to a hazardous act.

2		DEMOLITION

a)	Before commencing the work of demolition of a building or structure, all gas, electric, water and other meters shall be removed and the supply lines disconnected, except such as are especially provided or required for use in connection with the work of demolition.

b)	Glazed sashes and glazed doors shall be removed before the start of demolition operations.

c)	No wall or other construction shall be allowed to fall in mass, except under competent supervision. Scaffolds or staging shall be erected for workers if walls or other elements of the structure are too thin or too weak to work on.

d)	Stairs and stair railings shall be kept in place and in usable condition as long as is practicable, and steps and landings shall be kept from debris.

e)	Floor openings, unless covered or otherwise protected, shall be provided with guard rails and toe boards.

f)	All areas of danger in demolition operations shall be properly enclosed and danger signs posted.  Sufficient watchmen shall be provided to warn workers of impending dangers, and all unauthorised persons shall be excluded from places where demolition is in progress.

3		EXCAVATION

a)	The sides of every excavation in connection with building operations, including trenches for pipes or for any purposes, shall be sheet-piled, braced or shored when necessary to prevent the soil from caving in on persons engaged in work within such excavation.

b)	Where workers are employed adjacent to an excavation on work other than that directly connected with the excavation, substantial railings or fences shall be provided to prevent such workers from falling into the excavation.

- Every trench, five feet or more in depth, shall have suitable means of exit or escape at least every 25 feet of its length.

d)	Excavations shall be drained and kept clear of excess water and debris to the satisfaction of the Authority.

4		CONSTRUCTION FENCES

Buildings which are erected or demolished closer than five feet to a street line and which are not required to have a sidewalk shed shall be provided with protection on the street sides in the form of a substantial fence not less than eight feet high.  Such fence shall not restrict the sidewalk to less than five feet in width and shall be built solid for its full length, except for such openings as may be necessary for a proper execution of the work.

The Director may waive, or may vary, the provisions of subsection 4 where the safety of the public may otherwise be protected.

5		STORAGE OF MATERIAL

5.1		General

a)	Materials to be stored at or near locations where workers are employed or on any public property shall be piled or stacked in an orderly manner to avoid toppling over or being otherwise displaced.

b)	No material shall be piled or stacked to a greater height than six feet except in yards or sheds intended especially for storage.  When piles exceed four feet in height, the material shall be so placed that the sides and ends of the piles taper back.

c)	The placing of construction materials in a building or structure during building operations shall be done with due consideration of the effect of such loads on the structural members, and such loads shall, in general, be placed as near to the points of support of the structural members as possible.

d)	Waste material and other rubbish resulting from building operations shall be removed as often as possible and shall not be allowed to accumulate on the premises or adjacent thereto.

5.2		Hazardous Materials

a)	When storing hazardous materials (e.g. flammable, explosive or toxic materials) the manufacturer's recommendations shall be followed. In any event the owner and builder shall construct and maintain safe storage areas in accordance with the applicable laws of Grenada.

b)	In general hazardous materials should be stored in well ventilated, shaded areas away from naked flames, water courses and high traffic areas.

6		LADDERS

a)	All ladders, when in use, shall be set -up in a manner to be secure and to prevent slipping. Ladders, except stepladders or other self-supporting ladders, shall be securely fastened to a permanent support at the top, and if necessary, at the bottom, and braced to prevent swaying, bending or shaking.

b)	Ladders, leading to a floors, or platforms, should extend at last three feet above the level of such floor, or platform.

c)	A single ladder should not exceed 20 feet in length.

d)	Ladder rungs shall be spaced uniformly as near to 12 inches as is practicable.

e)	Ladders, other than sectional or extension ladders, shall not be extended by joining two or more together

f)	A ladder should be inspected by the builder before being put to use on a building operation and thereafter at least once every 30 days while continued in use.  Broken or weak ladders with weak or missing rungs, shall not be used or permitted to remain on the site of building operations, but shall be repaired and made safe or destroyed.

g)	The angle of any access ladder shall be at not less than 60 degrees and not more than 80 degrees to the horizontal.

7		SCAFFOLDS

a)	Properly constructed scaffolds shall be provided for all work that cannot be done safely by workmen standing on permanent or solid construction, except when such work can be done safely from ladders. All such scaffolds shall be substantially constructed, to support at least four times the maximum load and shall be secured to prevent swaying.

b)	Planks used in the construction of stationary scaffolds shall be not less than two inches nominal thickness.  Where such planks overlap at the ends, the overlap shall be not less than six inches.

c)	Planks shall be so placed that they cannot tip under the weight of the worker at any point.  Nails used in the construction of scaffolds shall be of ample size and length to carry the loads they are intended to support and all nails shall be driven full length.  No nails shall be subject to direct pull.

d)	Every scaffold, the platform level of which is more than six feet above the ground or above a permanent or temporary floor, other than iron workers' scaffolds and carpenters' bracket scaffolds, shall be provided with guard rails and toe board extending the full length of the scaffold.

e)	At doorways, passageways or other points where workers must pass under scaffolds, a substantial overhead protection shall be provided. No materials or equipment, other than required by the workers, shall be placed on scaffold platforms.

f)	Roof brackets, roof scantling, crawling boards and similar forms of support shall be substantial in construction and securely fastened in place when in use.

g)	Barrels, boxes or other similar unstable objects shall not be used as supports for planking intended as scaffolds or places of work.

h)	When used over public sidewalks or other places of public use, scaffolds used for minor building repairs, alterations, or painting shall be equipped with drop cloths to effectively prevent the falling of paint or debris.

8		TEMPORARY LIGHT AND POWER

a)	Temporary electric service poles shall be self-supporting or adequately braced or guyed at all times.

b)	The installation of temporary lighting and power must be done in accordance with the electricity supply regulations of the State. The developer must obtain a permit for this installation from the Electrical Inspector before electricity can be supplied to the site.

9		SANITATION

a)	Adequate toilet facilities, maintained in a clean, sanitary condition should be accessible to workers.

b)	An adequate supply of pure, drinking water shall be provided for workers during hours of employment, and adequate, sanitary washing facilities shall be provided for workers within reasonable access.

10		OPEN FIRES

a)	Open fires, for the purposes of disposing of waste materials, the heating of roofing or other materials, or for any other purpose whatsoever, shall not be allowed except with the permission of the Fire Services Department.

b)	Wherever any enclosed flame heaters or open fires are used, there shall be a workman in constant attendance, whose duty it shall be to have such heater or fire under proper control at all times.

11		SPECIAL HURRICANE PRECAUTIONS

a)	During  periods declared by the Government as a hurricane watch, all construction materials or equipment shall be secured against displacement by wind forces; provided that where a full complement of personnel is employed or otherwise in attendance, or engaged for such protection purposes, normal construction procedures or use of materials or equipment may continue but allowing such reasonable times as may be necessary to secure such materials or equipment before winds of hurricane force are anticipated.

b)	Construction materials and equipment shall be secured by guying and shoring, and by tying down loose materials, equipment and construction sheds.