APPENDIX III: SECTION 2.0

MODEL PARAMETERS FOR BUILDINGS AND CONTENTS IN ST. LUCIA

Table A3-1. Market Buildings in St. Lucia

(Building Type SL-1: 1 – 3 Story Commercial Masonry)*

| Building Element | Description | # Model Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | a1 (mph)** | a2 (mph)** | b1 | b2 | II | JI |
| Roof Covering | Metal Sheathing | 58 | 185 | 0.05 | 3.00 | 3.00 | 3.00 |
| Roof Decking | Plywood | 55 | 176 | 0.05 | 3.00 | 3.00 | 3.00 |
| Roof Framing | Wooden Truss | 99 | 220 | 0.05 | 3.00 | 3.00 | 3.00 |
| Roof-wall Connection | Hurricane Straps | 76 | 185 | 0.50 | 2.05 | 3.00 | 3.00 |
| Roof-wall (and Intp.) | Hurricane Straps | 69 | 177 | 0.50 | 2.05 | 3.00 | 3.00 |
| Lateral Bracing | RC Masonry | 104 | 231 | 0.50 | 2.05 | 0.10 | 0.00 |
| Openings | Not Protected | 55 | 187 | 0.05 | 3.00 | 3.00 | 3.00 |
| Cladding | RC Masonry | 104 | 231 | 0.05 | 3.00 | 0.10 | 0.00 |
| FF Connection | Rigid | 132 | 220 | 0.50 | 2.05 | 0.10 | 0.00 |
| Foundation | Grade Beam | 132 | 275 | 0.50 | 2.05 | 0.10 | 0.00 |
| Gross Structure | N/A | 104 | 275 | 0.50 | 2.05 | 0.00 | 0.40 |

*    Main features			** 1-minute sustained

- Fully engineered

- 1990 construction

- Average construction quality

- Hip roof

Table A3-2. Commercial Buildings in St. Lucia

(Building Type SL-2: 1 – 3 Story Commercial Masonry)*

| Building Element | Description | Model Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | a1 (mph)** | a2 (mph)** | b1 | b2 | II | JI |
| Roof Covering | MPS | 58 | 185 | 0.05 | 1.00 | 3.00 | 3.00 |
| Roof Decking | Wooden Plank | 55 | 176 | 0.05 | 1.00 | 3.00 | 3.00 |
| Roof Framing | Wooden Truss | 99 | 220 | 0.05 | 1.00 | 3.00 | 3.00 |
| Roof-wall Connection | Hurricane Straps | 76 | 185 | 0.40 | 1.00 | 3.00 | 3.00 |
| Roof-wall (and Intp.) | Hurricane Straps | 69 | 177 | 0.40 | 1.00 | 3.00 | 3.00 |
| Lateral Bracing | Shear Wall | 104 | 231 | 0.40 | 1.00 | 0.10 | 0.00 |
| Openings | Not Window Prot. | 55 | 187 | 0.05 | 1.00 | 3.00 | 3.00 |
| Cladding | RC Block | 104 | 231 | 0.05 | 1.00 | 0.10 | 0.00 |
| FF Connection | Rigid | 132 | 220 | 0.40 | 1.00 | 0.10 | 0.00 |
| Foundation | Grade Beam | 132 | 275 | 0.40 | 1.00 | 0.10 | 0.00 |
| Gross Structure | N/A | 104 | 275 | 0.40 | 1.00 | 0.00 | 0.40 |

*    Main features			** 1-minute sustained

- Fully engineered

- Hip or gable

Table A3-3. Commercial Buildings in St. Lucia

(Building Type SL-3: > 3 Story Commercial Masonry)*

| Building Element | Description | Model Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | a1 (mph)** | a2 (mph)** | b1 | b2 | II | JI |
| Roof Covering | Built-up | 58 | 185 | 0.05 | 2.00 | 3.00 | 3.00 |
| Roof Decking | CS | 74 | 210 | 0.05 | 2.00 | 3.00 | 0.10 |
| Roof Framing | RC | 74 | 210 | 0.05 | 2.00 | 3.00 | 0.10 |
| Roof-wall Connection | Rigid | 95 | 210 | 0.50 | 1.45 | 0.10 | 0.10 |
| Roof-wall (and Intp.) | Rigid | 95 | 210 | 0.50 | 1.45 | 0.10 | 0.10 |
| Lateral Bracing | RC | 100 | 220 | 0.50 | 1.45 | 0.10 | 0.10 |
| Openings | Not Protected | 60 | 200 | 0.05 | 2.00 | 3.00 | 3.00 |
| Cladding | RC | 100 | 220 | 0.05 | 2.00 | 0.10 | 0.10 |
| FF Connection | Yes | 160 | 250 | 0.50 | 1.45 | 0.10 | 0.10 |
| Foundation | Piles | 168 | 289 | 0.50 | 1.45 | 0.10 | 0.10 |
| Gross Structure | N/A | 100 | 289 | 0.50 | 1.45 | 0.00 | 0.40 |

*    Main features			** 1-minute sustained

- Fully engineered

- Average construction quality

- Flat roof

Table A3-4. Castries, Fisheries Complex in St. Lucia

(Building Type SL-4: 1 - 3 Story Commercial Masonry)*

| Building Element | Description | Model Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | a1 (mph)** | a2 (mph)** | b1 | b2 | II | JI |
| Roof Covering | MPS | 58 | 185 | 0.05 | 3.00 | 3.00 | 3.00 |
| Roof Decking | MD | 55 | 176 | 0.05 | 3.00 | 3.00 | 3.00 |
| Roof Framing | SB | 104 | 231 | 0.05 | 3.00 | 3.00 | 3.00 |
| Roof-wall Connection | Bolts | 76 | 185 | 0.50 | 2.05 | 3.00 | 3.00 |
| Roof-wall (and Intp.) | Bolts | 66 | 169 | 0.50 | 2.05 | 3.00 | 3.00 |
| Lateral Bracing | RC | 110 | 241 | 0.50 | 2.05 | 0.10 | 0.00 |
| Openings | Not Protected | 55 | 187 | 0.05 | 3.00 | 3.00 | 3.00 |
| Cladding | RC | 110 | 241 | 0.05 | 3.00 | 0.10 | 0.00 |
| FF Connection | Rigid | 132 | 220 | 0.50 | 2.05 | 0.10 | 0.00 |
| Foundation | Piles | 139 | 289 | 0.50 | 2.05 | 0.10 | 0.00 |
| Gross Structure | N/A | 110 | 289 | 0.50 | 2.05 | 0.00 | 0.40 |

*    Main features			** 1-minute sustained

- Fully engineered

- Average construction quality

- Gable roof

Table A3-5. Sports Complex in St. Lucia

(Building Type SL-5: 1 - 3 Story Commercial Masonry)*

| Building Element | Description | Model Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | a1 (mph)** | a2 (mph)** | b1 | b2 | II | JI |
| Roof Covering | MPS | 55 | 176 | 0.05 | 3.00 | 3.00 | 3.00 |
| Roof Decking | Plywood | 52 | 176 | 0.05 | 3.00 | 3.00 | 3.00 |
| Roof Framing | Wooden Truss | 94 | 209 | 0.05 | 3.00 | 3.00 | 3.00 |
| Roof-wall Connection | Hurricane Strap | 76 | 185 | 0.50 | 2.05 | 3.00 | 3.00 |
| Roof-wall (and Intp.) | Hurricane Strap | 66 | 169 | 0.50 | 2.05 | 3.00 | 3.00 |
| Lateral Bracing | Concrete Blocks | 100 | 241 | 0.50 | 2.05 | 0.10 | 0.00 |
| Openings | Not Protected | 55 | 187 | 0.05 | 3.00 | 3.00 | 3.00 |
| Cladding | Concrete Blocks | 100 | 241 | 0.05 | 3.00 | 0.10 | 0.00 |
| FF Connection | ? | 132 | 220 | 0.50 | 2.05 | 0.10 | 0.00 |
| Foundation | SF | 139 | 289 | 0.50 | 2.05 | 0.10 | 0.00 |
| Gross Structure | N/A | 100 | 289 | 0.50 | 2.05 | 0.00 | 0.40 |

*    Main features			** 1-minute sustained

- Fully engineered

- Constructed between 1980 and 1990

- Average construction quality

- Flat roof

Table A3-6. Marketing Board in St. Lucia

(Building Type SL-6: 1 - 3 Story Commercial Masonry)*

| Building Element | Description | Model Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | a1 (mph)** | a2 (mph)** | b1 | b2 | II | JI |
| Roof Covering | Built-up | 55 | 176 | 0.05 | 1.00 | 3.00 | 3.00 |
| Roof Decking | Concrete Deck | 55 | 176 | 0.05 | 1.00 | 3.00 | 3.00 |
| Roof Framing | Concrete | 55 | 176 | 0.05 | 1.00 | 3.00 | 3.00 |
| Roof-wall Connection | Rigid | 76 | 185 | 0.40 | 1.00 | 3.00 | 3.00 |
| Roof-wall (and Intp.) | Rigid | 69 | 177 | 0.40 | 1.00 | 3.00 | 3.00 |
| Lateral Bracing | Section Wall | 100 | 241 | 0.40 | 1.00 | 0.10 | 0.00 |
| Openings | Protected Windows | 85 | 196 | 0.05 | 1.00 | 3.00 | 3.00 |
| Cladding | RCB | 100 | 241 | 0.05 | 1.00 | 0.10 | 0.00 |
| FF Connection | Rigid | 132 | 220 | 0.40 | 1.00 | 0.10 | 0.00 |
| Foundation | Piles | 139 | 289 | 0.40 | 1.00 | 0.10 | 0.00 |
| Gross Structure | N/A | 100 | 289 | 0.40 | 1.00 | 0.00 | 0.40 |

*    Main features			** 1-minute sustained

- Fully engineered

- 1970 construction

- Flat roof

Table A3-7. Water & Sewerage in St. Lucia

(Building Type SL-7)*

| Building Element | Description | Model Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | a1 (mph)** | a2 (mph)** | b1 | b2 | II | JI |
| Roof Covering | MPS | 58 | 185 | 0.05 | 1.00 | 3.00 | 3.00 |
| Roof Decking | Wooden Plank | 55 | 176 | 0.05 | 1.00 | 3.00 | 3.00 |
| Roof Framing | Wooden Truss | 99 | 220 | 0.05 | 1.00 | 3.00 | 3.00 |
| Roof-wall Connection | Straps | 76 | 185 | 0.40 | 1.00 | 3.00 | 3.00 |
| Roof-wall (and Intp.) | Straps | 69 | 177 | 0.40 | 1.00 | 3.00 | 3.00 |
| Lateral Bracing | RC | 104 | 231 | 0.40 | 1.00 | 0.10 | 0.00 |
| Openings | No Protection | 55 | 187 | 0.05 | 1.00 | 3.00 | 3.00 |
| Cladding | RCB | 104 | 177 | 0.05 | 1.00 | 0.10 | 0.00 |
| FF Connection | Rigid | 132 | 220 | 0.40 | 1.00 | 0.10 | 0.00 |
| Foundation | SF | 139 | 289 | 0.40 | 1.00 | 0.10 | 0.00 |
| Gross Structure | N/A | 104 | 289 | 0.40 | 1.00 | 0.00 | 0.40 |

*    Main features			** 1-minute sustained

- Fully engineered

- 1970 construction

- Gable

Table A3-8. Commercial Buildings Owned by Government in St. Lucia

(Building Type SL-8: 1 - 3 Story Commercial Masonry)*

| Building Element | Description | Model Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | a1 (mph)** | a2 (mph)** | b1 | b2 | II | JI |
| Roof Covering | MPS | 58 | 185 | 0.05 | 2.00 | 3.00 | 3.00 |
| Roof Decking | Metal Deck | 55 | 176 | 0.05 | 2.00 | 3.00 | 3.00 |
| Roof Framing | Steel Beam | 104 | 231 | 0.05 | 2.00 | 3.00 | 3.00 |
| Roof-wall Connection | Bolts | 76 | 185 | 0.50 | 1.45 | 3.00 | 3.00 |
| Roof-wall (and Intp.) | Bolts | 66 | 169 | 0.50 | 1.45 | 3.00 | 3.00 |
| Lateral Bracing | Steel | 104 | 231 | 0.50 | 1.45 | 0.10 | 0.00 |
| Openings | Not Protected | 55 | 187 | 0.05 | 2.00 | 3.00 | 3.00 |
| Cladding | RCB | 110 | 241 | 0.05 | 2.00 | 0.10 | 0.00 |
| FF Connection | Yes | 132 | 220 | 0.50 | 1.45 | 0.10 | 0.00 |
| Foundation | Spread Footing (SF) | 139 | 289 | 0.50 | 1.45 | 0.10 | 0.00 |
| Gross Structure | N/A | 104 | 289 | 0.50 | 1.45 | 0.00 | 0.40 |

*    Main features			** 1-minute sustained

- Fully engineered

- Constructed between 1972 and 1988

- Gable roof

- Average construction quality

Table A3-9. Schools in St. Lucia

(Building Type SL-9: 1 - 3 Story Commercial Wooden)*

| Building Element | Description | Model Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | a1 (mph)** | a2 (mph)** | b1 | b2 | II | JI |
| Roof Covering | MPS | 58 | 185 | 0.05 | 1.00 | 3.00 | 3.00 |
| Roof Decking | Wooden Planks | 55 | 176 | 0.05 | 1.00 | 3.00 | 3.00 |
| Roof Framing | Wooden Truss | 99 | 220 | 0.05 | 1.00 | 3.00 | 3.00 |
| Roof-wall Connection | Straps | 76 | 185 | 0.40 | 1.00 | 3.00 | 3.00 |
| Roof-wall (and Intp.) | Straps | 69 | 177 | 0.40 | 1.00 | 3.00 | 3.00 |
| Lateral Bracing | Steel or Wood | 94 | 209 | 0.40 | 1.00 | 0.10 | 0.00 |
| Openings | Not Protected | 55 | 187 | 0.05 | 1.00 | 3.00 | 3.00 |
| Cladding | RM/ Wood | 55 | 220 | 0.05 | 1.00 | 0.10 | 0.00 |
| FF Connection | Straps | 125 | 209 | 0.40 | 1.00 | 0.10 | 0.00 |
| Foundation | Blocks | 132 | 275 | 0.40 | 1.00 | 0.10 | 0.00 |
| Gross Structure | N/A | 132 | 275 | 0.40 | 1.00 | 0.00 | 0.40 |

*    Main features			** 1-minute sustained

- Fully engineered

- Constructed between 1960 and 1990

- Gable roof

- Average construction quality

Table A3-10. Airport Buildings in St. Lucia

(Building Type SL-10)*

| Building Element | Description | Model Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | a1 (mph)** | a2 (mph)** | b1 | b2 | II | JI |
| Roof Covering | MPS | 58 | 185 | 0.00 | 0.50 | 3.00 | 3.00 |
| Roof Decking | Metal | 55 | 176 | 0.00 | 0.50 | 3.00 | 3.00 |
| Roof Framing | Steel/ Wooden Truss | 99 | 220 | 0.00 | 0.50 | 3.00 | 3.00 |
| Roof-wall Connection | Yes | 76 | 185 | 0.25 | 1.00 | 3.00 | 3.00 |
| Roof-wall (and Intp.) | Yes | 69 | 177 | 0.25 | 1.00 | 3.00 | 3.00 |
| Lateral Bracing | Steel/ RM | 104 | 231 | 0.25 | 1.00 | 0.10 | 0.00 |
| Openings | Not Protected | 55 | 187 | 0.00 | 0.50 | 3.00 | 3.00 |
| Cladding | RM | 104 | 231 | 0.00 | 0.50 | 0.10 | 0.00 |
| FF Connection | Yes | 132 | 220 | 0.25 | 1.00 | 0.10 | 0.00 |
| Foundation | Piles | 132 | 275 | 0.25 | 1.00 | 0.10 | 0.00 |
| Gross Structure | N/A | 104 | 275 | 0.25 | 1.00 | 0.00 | 0.40 |

*    Main features			** 1-minute sustained

- Fully engineered

- 1 – 3 stories

- Gable roof

Table A3-11. Victoria Hospital in St. Lucia

(Building Type SL-11: 1 –3 Story Masonry)*

| Building Element | Description | Model Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | a1 (mph)** | a2 (mph)** | b1 | b2 | II | JI |
| Roof Covering | MPS | 58 | 185 | 0.00 | 0.50 | 3.00 | 3.00 |
| Roof Decking | WP | 55 | 176 | 0.00 | 0.50 | 3.00 | 3.00 |
| Roof Framing | WT | 99 | 220 | 0.00 | 0.50 | 3.00 | 3.00 |
| Roof-wall Connection | RC/ Wood | 76 | 185 | 0.25 | 1.00 | 3.00 | 3.00 |
| Roof-wall (and Intp.) | RC/ Wood | 69 | 177 | 0.25 | 1.00 | 3.00 | 3.00 |
| Lateral Bracing | RC | 104 | 231 | 0.25 | 1.00 | 0.10 | 0.00 |
| Openings | Not Protected | 55 | 187 | 0.00 | 0.50 | 3.00 | 3.00 |
| Cladding | RM | 104 | 231 | 0.00 | 0.50 | 0.10 | 0.00 |
| FF Connection | Yes | 132 | 220 | 0.25 | 1.00 | 0.10 | 0.00 |
| Foundation | Blocks/SOG | 132 | 275 | 0.25 | 1.00 | 0.10 | 0.00 |
| Gross Structure | N/A | 104 | 275 | 0.25 | 1.00 | 0.00 | 0.40 |

*    Main features			** 1-minute sustained

- Fully engineered

- 1 – 3 stories

- Average construction quality

- Constructed between 1960 and 1990

APPENDIX III: SECTION 3.0

VULNERABILITY CURVES FOR ST. LUCIA BUILDINGS

Table A3-12. Market Buildings in St. Lucia

(Building Type SL-1: 1 – 3 Story Commercial Masonry)

| Wind Speed (mph) | Structural Damage Ratio | Content Damage Ratio |
| --- | --- | --- |
| 50 | 0.000000 | 0.000000 |
| 60 | 0.001106 | 0.000000 |
| 70 | 0.012161 | 0.000000 |
| 80 | 0.039224 | 0.000068 |
| 90 | 0.088620 | 0.001046 |
| 100 | 0.161360 | 0.004238 |
| 110 | 0.260120 | 0.011304 |
| 120 | 0.386002 | 0.024109 |
| 130 | 0.523675 | 0.043474 |
| 140 | 0.649530 | 0.073061 |
| 150 | 0.757181 | 0.109279 |
| 160 | 0.846611 | 0.144582 |
| 170 | 0.912924 | 0.172486 |
| 180 | 0.952294 | 0.188174 |
| 190 | 0.972779 | 0.194947 |
| 200 | 0.986257 | 0.199595 |
| 210 | 0.994792 | 0.202973 |
| 220 | 0.998229 | 0.204687 |
| 230 | 0.998922 | 0.205498 |
| 240 | 0.999349 | 0.206268 |
| 250 | 0.999668 | 0.206916 |
| 260 | 0.999880 | 0.207383 |
| 270 | 0.999987 | 0.207626 |
| 280 | 1.000000 | 0.207657 |
| 290 | 1.000000 | 0.207657 |
| 300 | 1.000000 | 0.207657 |

Table A3-13. Commercial Buildings in St. Lucia

(Building Type SL-2: 1 – 3 Story Commercial Masonry)

| Wind Speed (mph) | Structural Damage Ratio | Content Damage Ratio |
| --- | --- | --- |
| 50 | 0.000000 | 0.000000 |
| 60 | 0.001106 | 0.000000 |
| 70 | 0.012161 | 0.000000 |
| 80 | 0.039224 | 0.000659 |
| 90 | 0.088620 | 0.010082 |
| 100 | 0.161360 | 0.040869 |
| 110 | 0.260120 | 0.108997 |
| 120 | 0.386002 | 0.232821 |
| 130 | 0.523675 | 0.412051 |
| 140 | 0.649530 | 0.617403 |
| 150 | 0.757181 | 0.775885 |
| 160 | 0.846611 | 0.873692 |
| 170 | 0.912924 | 0.934071 |
| 180 | 0.952294 | 0.960970 |
| 190 | 0.972779 | 0.974152 |
| 200 | 0.986257 | 0.982779 |
| 210 | 0.994792 | 0.989847 |
| 220 | 0.998229 | 0.994830 |
| 230 | 0.998922 | 0.997683 |
| 240 | 0.999349 | 0.999152 |
| 250 | 0.999668 | 0.999779 |
| 260 | 0.999880 | 0.999971 |
| 270 | 0.999987 | 1.000000 |
| 280 | 1.000000 | 1.000000 |
| 290 | 1.000000 | 1.000000 |
| 300 | 1.000000 | 1.000000 |

Table A3-14. Commercial Buildings in St. Lucia

(Building Type SL-3: > 3 Story Commercial Masonry)

| Wind Speed (mph) | Structural Damage Ratio | Content Damage Ratio |
| --- | --- | --- |
| 50 | 0.000000 | 0.000000 |
| 60 | 0.000118 | 0.000000 |
| 70 | 0.006681 | 0.000000 |
| 80 | 0.025861 | 0.000022 |
| 90 | 0.065280 | 0.001682 |
| 100 | 0.125821 | 0.009059 |
| 110 | 0.208064 | 0.027176 |
| 120 | 0.312291 | 0.062432 |
| 130 | 0.434234 | 0.118073 |
| 140 | 0.561625 | 0.184254 |
| 150 | 0.682880 | 0.250658 |
| 160 | 0.784136 | 0.311622 |
| 170 | 0.864080 | 0.364026 |
| 180 | 0.922581 | 0.402672 |
| 190 | 0.960445 | 0.424551 |
| 200 | 0.982101 | 0.435238 |
| 210 | 0.990620 | 0.440868 |
| 220 | 0.993231 | 0.447404 |
| 230 | 0.995442 | 0.454754 |
| 240 | 0.997201 | 0.462638 |
| 250 | 0.998351 | 0.469792 |
| 260 | 0.999088 | 0.475796 |
| 270 | 0.999608 | 0.480563 |
| 280 | 0.999912 | 0.483442 |
| 290 | 1.000000 | 0.484258 |
| 300 | 1.000000 | 0.484258 |

Table A3-15. Castries, Fisheries Complex in St. Lucia

(Building Type SL-4: 1 - 3 Story Commercial Masonry)

| Wind Speed (mph) | Structural Damage Ratio | Content Damage Ratio |
| --- | --- | --- |
| 50 | 0.000000 | 0.000000 |
| 60 | 0.001106 | 0.000000 |
| 70 | 0.012625 | 0.000000 |
| 80 | 0.041866 | 0.000068 |
| 90 | 0.093996 | 0.001046 |
| 100 | 0.170003 | 0.004238 |
| 110 | 0.270616 | 0.011304 |
| 120 | 0.397666 | 0.024410 |
| 130 | 0.530540 | 0.047447 |
| 140 | 0.650075 | 0.078585 |
| 150 | 0.750377 | 0.113713 |
| 160 | 0.831475 | 0.145368 |
| 170 | 0.893149 | 0.168688 |
| 180 | 0.933724 | 0.182880 |
| 190 | 0.957278 | 0.189921 |
| 200 | 0.974243 | 0.195181 |
| 210 | 0.986727 | 0.199572 |
| 220 | 0.994695 | 0.202817 |
| 230 | 0.998145 | 0.204570 |
| 240 | 0.998839 | 0.205351 |
| 250 | 0.999265 | 0.206102 |
| 260 | 0.999594 | 0.206757 |
| 270 | 0.999826 | 0.207258 |
| 280 | 0.999961 | 0.207566 |
| 290 | 1.000000 | 0.207657 |
| 300 | 1.000000 | 0.207657 |

Table A3-16. Sports Complex in St. Lucia

(Building Type SL-5: 1 - 3 Story Commercial Masonry)

| Wind Speed (mph) | Structural Damage Ratio | Content Damage Ratio |
| --- | --- | --- |
| 50 | 0.000000 | 0.000000 |
| 60 | 0.002382 | 0.000000 |
| 70 | 0.016585 | 0.000000 |
| 80 | 0.048707 | 0.000166 |
| 90 | 0.103917 | 0.001535 |
| 100 | 0.184089 | 0.005508 |
| 110 | 0.292983 | 0.013882 |
| 120 | 0.428290 | 0.028702 |
| 130 | 0.565588 | 0.052879 |
| 140 | 0.690112 | 0.085435 |
| 150 | 0.796058 | 0.122747 |
| 160 | 0.879892 | 0.156936 |
| 170 | 0.935458 | 0.180791 |
| 180 | 0.966723 | 0.193716 |
| 190 | 0.982813 | 0.199358 |
| 200 | 0.991966 | 0.202324 |
| 210 | 0.995809 | 0.203543 |
| 220 | 0.997218 | 0.204119 |
| 230 | 0.998186 | 0.204817 |
| 240 | 0.998839 | 0.205551 |
| 250 | 0.999265 | 0.206247 |
| 260 | 0.999594 | 0.206845 |
| 270 | 0.999826 | 0.207298 |
| 280 | 0.999961 | 0.207575 |
| 290 | 1.000000 | 0.207657 |
| 300 | 1.000000 | 0.207657 |

Table A3-17. Marketing Board in St. Lucia

(Building Type SL-6: 1 - 3 Story Commercial Masonry)

| Wind Speed (mph) | Structural Damage Ratio | Content Damage Ratio |
| --- | --- | --- |
| 50 | 0.000000 | 0.000000 |
| 60 | 0.001670 | 0.000000 |
| 70 | 0.015062 | 0.000000 |
| 80 | 0.045582 | 0.001357 |
| 90 | 0.100220 | 0.014924 |
| 100 | 0.183934 | 0.055668 |
| 110 | 0.297493 | 0.143963 |
| 120 | 0.438303 | 0.300131 |
| 130 | 0.583644 | 0.478443 |
| 140 | 0.716671 | 0.699386 |
| 150 | 0.826336 | 0.873720 |
| 160 | 0.906872 | 0.948694 |
| 170 | 0.958268 | 0.972521 |
| 180 | 0.981601 | 0.977621 |
| 190 | 0.990762 | 0.978594 |
| 200 | 0.993963 | 0.981219 |
| 210 | 0.995809 | 0.985844 |
| 220 | 0.997218 | 0.991418 |
| 230 | 0.998186 | 0.995412 |
| 240 | 0.998839 | 0.997818 |
| 250 | 0.999265 | 0.999124 |
| 260 | 0.999594 | 0.999732 |
| 270 | 0.999826 | 0.999951 |
| 280 | 0.999961 | 0.999998 |
| 290 | 1.000000 | 1.000000 |
| 300 | 1.000000 | 1.000000 |

Table A3-18. Water & Sewerage in St. Lucia (Building Type SL-7)

| Wind Speed (mph) | Structural Damage Ratio | Content Damage Ratio |
| --- | --- | --- |
| 50 | 0.000000 | 0.000000 |
| 60 | 0.001106 | 0.000000 |
| 70 | 0.012161 | 0.000000 |
| 80 | 0.039224 | 0.000659 |
| 90 | 0.088620 | 0.010082 |
| 100 | 0.161360 | 0.040869 |
| 110 | 0.260169 | 0.108997 |
| 120 | 0.386352 | 0.232821 |
| 130 | 0.524598 | 0.412051 |
| 140 | 0.651267 | 0.617403 |
| 150 | 0.759590 | 0.775885 |
| 160 | 0.849139 | 0.873692 |
| 170 | 0.915029 | 0.934071 |
| 180 | 0.953635 | 0.960970 |
| 190 | 0.973381 | 0.972925 |
| 200 | 0.986245 | 0.979453 |
| 210 | 0.994336 | 0.984880 |
| 220 | 0.997618 | 0.990652 |
| 230 | 0.998318 | 0.995003 |
| 240 | 0.998840 | 0.997623 |
| 250 | 0.999265 | 0.999046 |
| 260 | 0.999594 | 0.999708 |
| 270 | 0.999826 | 0.999946 |
| 280 | 0.999961 | 0.999997 |
| 290 | 1.000000 | 1.000000 |
| 300 | 1.000000 | 1.000000 |

Table A3-19. Commercial Buildings Owned by Government in St. Lucia

(Building Type SL-8: 1 - 3 Story Commercial Masonry)

| Wind Speed (mph) | Structural Damage Ratio | Content Damage Ratio |
| --- | --- | --- |
| 50 | 0.000000 | 0.000000 |
| 60 | 0.001106 | 0.000000 |
| 70 | 0.012625 | 0.000000 |
| 80 | 0.041866 | 0.000156 |
| 90 | 0.093996 | 0.002393 |
| 100 | 0.170003 | 0.009700 |
| 110 | 0.270640 | 0.025870 |
| 120 | 0.397775 | 0.055978 |
| 130 | 0.530742 | 0.110894 |
| 140 | 0.650379 | 0.187045 |
| 150 | 0.750790 | 0.274079 |
| 160 | 0.832005 | 0.352434 |
| 170 | 0.893795 | 0.409106 |
| 180 | 0.934328 | 0.442910 |
| 190 | 0.957793 | 0.459096 |
| 200 | 0.974660 | 0.471201 |
| 210 | 0.987038 | 0.481562 |
| 220 | 0.994893 | 0.489277 |
| 230 | 0.998221 | 0.493551 |
| 240 | 0.998839 | 0.495566 |
| 250 | 0.999265 | 0.497476 |
| 260 | 0.999594 | 0.499126 |
| 270 | 0.999826 | 0.500381 |
| 280 | 0.999961 | 0.501111 |
| 290 | 1.000000 | 0.501317 |
| 300 | 1.000000 | 0.501317 |

Table A3-20. Schools in St. Lucia

(Building Type SL-9: 1 - 3 Story Commercial Wooden)

| Wind Speed (mph) | Structural Damage Ratio | Content Damage Ratio |
| --- | --- | --- |
| 50 | 0.000000 | 0.000000 |
| 60 | 0.001116 | 0.000000 |
| 70 | 0.012251 | 0.000000 |
| 80 | 0.039474 | 0.000659 |
| 90 | 0.089109 | 0.010082 |
| 100 | 0.162198 | 0.040869 |
| 110 | 0.261490 | 0.108997 |
| 120 | 0.387899 | 0.232821 |
| 130 | 0.526113 | 0.412051 |
| 140 | 0.652659 | 0.617403 |
| 150 | 0.760893 | 0.775885 |
| 160 | 0.850630 | 0.873692 |
| 170 | 0.916757 | 0.934071 |
| 180 | 0.955420 | 0.960970 |
| 190 | 0.975096 | 0.972800 |
| 200 | 0.987763 | 0.977512 |
| 210 | 0.995487 | 0.982407 |
| 220 | 0.998392 | 0.989429 |
| 230 | 0.998924 | 0.995263 |
| 240 | 0.999349 | 0.998266 |
| 250 | 0.999668 | 0.999549 |
| 260 | 0.999880 | 0.999942 |
| 270 | 0.999987 | 0.999999 |
| 280 | 1.000000 | 1.000000 |
| 290 | 1.000000 | 1.000000 |
| 300 | 1.000000 | 1.000000 |

Table A3-21. Airport Buildings in St. Lucia (Building Type SL-10)

| Wind Speed (mph) | Structural Damage Ratio | Content Damage Ratio |
| --- | --- | --- |
| 50 | 0.000000 | 0.000000 |
| 60 | 0.001106 | 0.000026 |
| 70 | 0.012161 | 0.002518 |
| 80 | 0.039224 | 0.020919 |
| 90 | 0.088620 | 0.083343 |
| 100 | 0.161360 | 0.230837 |
| 110 | 0.260120 | 0.414684 |
| 120 | 0.386002 | 0.518465 |
| 130 | 0.523675 | 0.625132 |
| 140 | 0.649530 | 0.784690 |
| 150 | 0.757181 | 0.917243 |
| 160 | 0.846611 | 0.970420 |
| 170 | 0.912924 | 0.977566 |
| 180 | 0.952294 | 0.979877 |
| 190 | 0.972779 | 0.983320 |
| 200 | 0.986257 | 0.988574 |
| 210 | 0.994792 | 0.993545 |
| 220 | 0.998229 | 0.996691 |
| 230 | 0.998922 | 0.998517 |
| 240 | 0.999349 | 0.999457 |
| 250 | 0.999668 | 0.999859 |
| 260 | 0.999880 | 0.999982 |
| 270 | 0.999987 | 1.000000 |
| 280 | 1.000000 | 1.000000 |
| 290 | 1.000000 | 1.000000 |
| 300 | 1.000000 | 1.000000 |

Table A3-22. Victoria Hospital in St. Lucia

(Building Type SL-11: 1 –3 Story Masonry)

| Wind Speed (mph) | Structural Damage Ratio | Content Damage Ratio |
| --- | --- | --- |
| 50 | 0.000000 | 0.000000 |
| 60 | 0.001106 | 0.000026 |
| 70 | 0.012161 | 0.002518 |
| 80 | 0.039224 | 0.020919 |
| 90 | 0.088620 | 0.083343 |
| 100 | 0.161360 | 0.230837 |
| 110 | 0.260120 | 0.414684 |
| 120 | 0.386002 | 0.518465 |
| 130 | 0.523675 | 0.625132 |
| 140 | 0.649530 | 0.784690 |
| 150 | 0.757181 | 0.917243 |
| 160 | 0.846611 | 0.970420 |
| 170 | 0.912924 | 0.977566 |
| 180 | 0.952294 | 0.979877 |
| 190 | 0.972779 | 0.983320 |
| 200 | 0.986257 | 0.988574 |
| 210 | 0.994792 | 0.993545 |
| 220 | 0.998229 | 0.996691 |
| 230 | 0.998922 | 0.998517 |
| 240 | 0.999349 | 0.999457 |
| 250 | 0.999668 | 0.999859 |
| 260 | 0.999880 | 0.999982 |
| 270 | 0.999987 | 1.000000 |
| 280 | 1.000000 | 1.000000 |
| 290 | 1.000000 | 1.000000 |
| 300 | 1.000000 | 1.000000 |

APPENDIX III: SECTION 4.0

SUMMARY OF PML ANALYSIS FOR ST. LUCIA